Document details

Title 	ÖBB DOSTO Neu HW & SW Release Note (Internal)

Purpose	Trace IOB System HW & SW evolution

Issued to	Nomad Digital (internal)

Document Reference	ND-DEL-OBB-035-RN-001-01-INT <TBD – confirm document number per BMS-ENGI-INS-001>

BMS Document Reference	BMS-ENGI-FOR-006-02

BMS Version Number 	01

Last updated 	06 July 2026

Author 	Abbas Rizvi

Approver	<TBD – approver name>

Number of pages 	<auto – update on final>

Related documents 	ND-DEL-OBB-035-SDD-002-01 (System Design Document); ND-BID-OBB-036 (Technical Description); Nomad Connect DEVOPS Test Exit Report

This document and any related documents contain strictly confidential information intended only 
for the designated recipient(s). Any unauthorised disclosure, dissemination, distribution or copying of this 
document or any related documents in any form is strictly prohibited.

INTERNAL — Nomad Digital only. Not for issue to ÖBB.

Document control

| Version | Revision date | Summary of edits | Created By | Approved By |
| --- | --- | --- | --- | --- |
| 1 | 05 July 2026 | First version of the document | Abbas Rizvi | <TBD> |
| 2 | 06 July 2026 | Per-material HW/FW/configuration paragraphs (CCU, Switch, AP), HW version before/after retrofit placeholders, and Common Software (Nomad Connect) section added per customer review (C. Catoire) | Abbas Rizvi | <TBD> |

Disclaimer of liability

The information in this document is subject to change without notice and should not be construed as a commitment by Nomad Digital Ltd. While reasonable precautions have been taken, Nomad Digital Ltd assumes no responsibility for technical inaccuracies or typographical errors that may appear in this document. Nomad Digital Ltd reserves the right to revise this publication without obligation to provide notification of such revisions.

Contents

Document control	1

Contents	2

1. Introduction	3

2. Hardware Platform 	3

3. Software Packages Delivered	3

4. Characteristics	3

1. Introduction

  - 1.1 Document Purpose

The purpose of this document is to describe the constitute components of the project software build developed by Nomad for deployment to ÖBB DOSTO Neu project’s Internet-on-Board (IoB) solution.

As part of Nomad’s quality assurance and change management processes, this document also provides a summary of the change log(s) and release history associated with the software release(s) through its evolution, with references made (where applicable) to Test Exit Report(s).

This covers the hardware and software deployed on the trains across all four DOSTO Neu platform variants: nv4 (4734, 4-car), nv6 (4736, 6-car), fv5 (4705, CAT 5-car) and fv6 (4706, FV 6-car).

The following documents give more details about the project:

| Title | Reference | Short Summary |
| --- | --- | --- |
| System Design Document (SDD) | ND-DEL-OBB-035-SDD-002-01 | Onboard network system design for the DOSTO Neu IoB solution |
| Nomad Connect DEVOPS TER report | ND-DEVOPS-OBB-TER-026-04 | Nomad Connect DEVOPS TEST EXIT REPORT |

  - Maturity Level

All current versions of software described in this version of the document are delivered for:

|  |  | Comments |
| --- | --- | --- |
|  | Tests on train |  |
| X | Pre-commercial service (deployment on delivered train) | v8 rollout in progress across the DOSTO Neu fleet (nv4, nv6, fv5, fv6) |
|  | Customer Test on train |  |
|  | Commercial service (fleet deployment) |  |

2. Hardware Platform Compatible With Delivered Software Release

This chapter mentions the current HW versions / Firmware of all the equipment part of ÖBB DOSTO Neu System when applicable compatible with delivered software release described in §3 of this document.

Each time HW Version / Firmware Version will change, the current versions will be updated in the “current version” paragraph and content of the evolutions will be clearly explained in the “HW Version evolution”.

  - List of Hardware

| NMID | Equipment description | Supplier | Comments |
| --- | --- | --- | --- |
| 1138 | Nomad Communication & Control Unit (CCU) – R5001C | Nomad Digital | 1 per trainset; onboard server / cellular gateway |
| 1143 | VDS Rail Consist Switch – VDSRail DS-CSWP28 (28-port, 10 GbE) | Nomad Digital | 3 per car (12 nv4 / 18 nv6 / 15 fv5 / 18 fv6) |
| 1043 | Westermo Ibex-610 Wi-Fi access point (RT610LV / IbexOS) | Nomad Digital | 4 per car (16 nv4 / 24 nv6 / ~20 fv5 / 24 fv6); passenger Wi-Fi |
| 1109 | 5G modem – Telit FN990A28 (LM990 carrier), integrated in CCU | Nomad Digital | Aggregated on CCU bond0 for WAN backhaul |
| 720 | GNSS / GPS receiver – uBlox NEO-M8L (integrated in CCU) | Nomad Digital | Passive – no firmware (NA) |
| N/A | Roof / Wi-Fi antennas – Sencity Rail Multi (1399.99.0307), Huber&Suhner indoor (1399.35.0020) | Stadler | Passive – no firmware (NA) |

The onboard network hardware above is common across all four DOSTO Neu platform variants; the switch and access-point counts and the switch configuration template differ per variant as follows:

| Platform | Series | Cars | Switches | Access Points | Configuration template |
| --- | --- | --- | --- | --- | --- |
| nv4 | 4734 | 4 | 12 | 16 | nd-obn-template-dostoneu-nv4 |
| nv6 | 4736 | 6 | 18 | 24 | nd-obn-template-dostoneu-nv6 |
| fv5 (CAT) | 4705 | 5 | 15 | ~20 | nd-obn-template-dostoneu-fv5 |
| fv6 (FV) | 4706 | 6 | 18 | 24 | nd-obn-template-dostoneu-fv6 |

  - System Description

  - 2.1 Nomad CCU (Communication & Control Unit – R5001C)

      - 2.1.1 Current Version

The Nomad CCU (NMID 1138) is the onboard server / cellular gateway, one per trainset. Its current baseline, common across all four DOSTO Neu platform variants (nv4, nv6, fv5, fv6), is:

- Hardware revision – before retrofit: <TBD>; after retrofit: <TBD> (to be confirmed with Tommy / Taram; read from the equipment label / asset register).

- Software – Nomad Connect platform 2025.2.1 (validated build). The onboard-network engine (nd-obn) 2.2.23 is embedded within Nomad Connect – see §2.4 Common Software.

- Configuration version – Nomad Connect configuration ID 7ce1d29ce6, with per-train parameters (Fzg-ID, IP addressing) applied at commissioning.

      - 2.1.2 HW Version Evolution

Hardware versions are read from the equipment labels. The before-retrofit and after-retrofit hardware versions are <TBD – to be confirmed with Tommy / Taram> and will be recorded here in the next issue of this document.

  - 2.2 VDS Rail Consist Switch (VDSRail DS-CSWP28)

      - 2.2.1 Current Version

The VDS Rail Consist Switch (NMID 1143) is the 28-port 10 GbE consist backbone switch, three per car. Its current baseline is:

- Hardware revision – before retrofit: <TBD>; after retrofit: <TBD> (to be confirmed with Tommy / Taram; read from the equipment label).

- Firmware – 7.4.2.

- Configuration version – V8 (per-platform template – nv4 / nv6 / fv5 / fv6 – with the per-train Fzg-ID rendered into switch hostnames).

      - 2.2.2 HW Version Evolution

Hardware versions are read from the equipment labels. The before-retrofit and after-retrofit hardware versions are <TBD – to be confirmed with Tommy / Taram> and will be recorded here in the next issue of this document.

  - 2.3 Westermo Wi-Fi Access Point (Ibex-610 / RT610LV)

      - 2.3.1 Current Version

The Westermo Ibex-610 (RT610LV) Wi-Fi access point (NMID 1043) provides passenger Wi-Fi, four per car. Its current baseline is:

- Hardware revision – before retrofit: <TBD>; after retrofit: <TBD> (to be confirmed with Tommy / Taram; read from the equipment label).

- Firmware – 6.11.2-0 (IbexOS).

- Configuration version – Nomad access-point configuration, delivered via the per-platform configuration template package (nd-obn-template-dostoneu-nv4 / nv6 / fv5 / fv6).

      - 2.3.2 HW Version Evolution

Hardware versions are read from the equipment labels. The before-retrofit and after-retrofit hardware versions are <TBD – to be confirmed with Tommy / Taram> and will be recorded here in the next issue of this document.

  - 2.4 Common Software – Nomad Connect (NC)

Nomad Connect (NC) is the common software platform running on the CCU. It manages the onboard network devices (switch and access-point discovery, configuration and firmware management) and is common across all four DOSTO Neu platform variants. The current common software baseline is:

- Nomad Connect platform – 2025.2.1 (validated build; configuration ID 7ce1d29ce6).

- Onboard-network engine (nd-obn) – 2.2.23. The OBN engine is embedded within Nomad Connect; it is listed here for traceability only, the Nomad Connect version being the release-defining version.

The individual software packages delivered are listed in §3.

3. Software Packages Delivered

  - This chapter mentions the current software release versions part of ÖBB DOSTO Neu delivery when delivered with compatible hardware as described in §2 of this document.

  - Each time software release will change, the current versions will be updated in the “current version” paragraph and content of the evolutions will be clearly explained in the “SW Version Evolution”.

| Delivered Software Table |  |  |  |
| --- | --- | --- | --- |
| SW# | SW Version# | Description |
| SW1 | 7.4.2 | VDS Rail Consist Switch firmware |
| SW2 | V8 | VDS Rail Consist Switch configuration (per-platform template – nv4 / nv6 / fv5 / fv6; per-train Fzg-ID) |
| SW3 | 6.11.2-0 | Westermo RT610LV Wi-Fi access point firmware (IbexOS) |
| SW4 | 2.2.23 | Onboard-network engine (nd-obn) on the CCU – embedded within Nomad Connect (SW5); listed for traceability only |
| SW5 | 2025.2.1 | CCU platform image (Nomad Connect base software) |

  - Software Testing Tools

| Delivered Software Table |  |  |
| --- | --- | --- |
| Package Name | Version | Description |
| Nomad NMS (Zabbix-based) | 2026.1.2 | Fleet network monitoring / SNMP polling used to verify device reachability and status |
| iperf3 | <TBD> | Throughput testing (TCP/UDP) during network validation |
| WiFiTestTool | <TBD> | Passenger Wi-Fi coverage / association testing |
| Ekahau | <TBD> | Wi-Fi survey / coverage measurement (where used) |

4. Characteristics

## New Functionalities

This is the first issue of the release note. It records the baseline onboard-network software and firmware deployed to the DOSTO Neu fleet: VDS Consist Switch configuration V8 with firmware 7.4.2, Westermo access-point firmware 6.11.2-0, and Nomad Connect (nd-obn) 2.2.23 on the CCU. No new functionality is introduced relative to a previous release note; subsequent updates will list added functions here with their reference (ticket / headline).

## Omissions and Restrictions

INTERNAL — the following known issues, workarounds and open items sit behind the customer release note. This list carries ticket references, bench and R&D detail and is NOT to be issued to ÖBB. The customer copy carries a reduced, non-ticketed subset of these.

- Westermo AP firmware stage-vs-activate flakiness. RT610LV (IbexOS) firmware pushes sometimes stage the image but do not activate it — (a) slow-but-fine (reboots late, self-recovers) or (b) genuine flash-trigger hang (rpcFwFlash=2, no reboot). OBN historically reported success on the SNMP SET echo without verifying activation; the bug-11 patch (westermo.py post-flash verify) makes reporting honest. Genuine hangs need a LuCI HTTPS upload or retry. Ref: TRIAG-8585 (OBN v8 hand-patch upstream, incl. bug 11). Surfaced in reduced form in the customer copy.

- OBN AP-firmware batch push silently fails — CCU TFTP conntrack-helper gap. obn update f ap batch pushes silently fail for most APs (only ~5 of ~15 succeed by race luck) because the CCU firewall lacks the nf_conntrack_tftp helper + udp/69 CT rule. Runtime workaround: modprobe nf_conntrack_tftp + iptables raw PREROUTING rule (wiped on every reboot). Durable fix pending in Puppet (60-allow-management). Ref: dosto-tftp-helper-check; TRIAG-8585 (infra item). Internal only.

- OBN coach-numbering drops bypassed switches — monitoring false-negative. When a VDS switch is cold-bypassed, OBN’s numbering walk mis-numbers the switch that moves into the gap, dead-ends, and normalise_devices() silently deletes every switch it could not number — including healthy, SNMP-reachable ones downstream, which then also miss the NMS report so nothing alarms. Fix: topology-anchored numbering + DOWN/UNPLACED rows (validated engine fix, prototyped on the bench). Cross-check OBN’s switch count against dosto-device-discovery. Ref: findings/RD_obn_coach_numbering_bypass_downstate_2026-07-04.md. Internal only.

- fv5 / fv6 empty obn report — backbone-discovery.yaml config drift. On some fv5/fv6 CCUs obn report returns empty because /etc/obn/backbone-discovery.yaml fell back to deb defaults (report_module: GenericReport, train_type: mq) instead of the DOSTO overrides. Fix per train: report_module: DostoNeuReport + correct train_type (nv4/nv6/fv5/fv6); enforce both in the Puppet template. Not a package-version issue. Internal only.

- Open cabling faults (Stadler-side). Per-train physical cabling faults (wrong neighbour, AP/switch not connected, inter-coach trunk PHY faults) are tracked in the cable-issues register — ~15 rows as of 2026-07-05, several OPEN pending Stadler. Outside Nomad’s scope. Ref: cable-issues-register.md / trackers/cable-issues-tracker.xlsx. Surfaced generically in the customer copy.

- Stadler-side firewall commissioning. vlan7 firewall commissioning is a Stadler activity; on some trains the FW is not yet commissioned (Westermo answers ARP but ICMP behaviour indicates policy not applied). Does not affect the Nomad onboard-network L2 baseline. Surfaced generically in the customer copy.

- box=Fzg / >127 octet limitation on 6-car, CAT and FV. factory up train-ID caps at 0-127 (3rd IP octet); Fzg for 4736/4706/4705 is 129-231, so box=Fzg only fits 4734 (nv4). The v9 templates’ remap-drop assumes train_id=Fzg; do not obn update c a CAT/FV train commissioned with box-id. Needs an R&D decision (fzg_id-key path). Internal only.

- R5001C / Telit FN990 modems stuck in REGISTERED after reboot / ignition power-cycle (mitigated). On the R5001C CCU the FN990 5G modems can stick in REGISTERED state (not CONNECTED) after a reboot or ignition power-cycle, leaving the CCU with fewer active WAN links until recovered. Mitigated on DOSTO by the nd-watcher-scripts modem watchdog (ships in Nomad Connect 2025.2.1 and 2025.4): after boot it detects any modem not reaching CONNECTED and automatically power-cycles it, so no manual intervention is required in service. Root cause is Telit FN990 modem firmware (CFUN=0/1 SIM-power handling); a modem firmware update alone did not fully resolve it, so the watchdog is the operative mitigation. Refs: DTR-6789 (Done), RD-12165, RD-11992, TRIAG-6692 (FN990 firmware). Confirmed working in service (Davud, 24 Jun 2026). Note: the same platform issue is open on the Avelia/Proxima fleet (TRIAG-8217, R5001C/FN990A40). Separately, a hardware fault has been observed on some CCUs where SIM slots fail to come up at boot, reducing redundant dual-SIM operation to a single active SIM; the remediation is modem carrier-blade replacement (some trains completed by SPL, some by Nomad, to bring them into commercial service). Cross-check against open Zabbix SIM/modem alarms (OBB Alpin, 6 Jul 2026): the recurring signature is a missing/inactive SIM on modem 0 together with modem 2 (not “the first two SIM slots” / modems 0 and 1 as sometimes stated) — the wording should be “two of the fitted SIMs” rather than a specific slot pair. A reliable fleet completion count cannot be derived from Zabbix: at any instant most CCUs are parked and report is_sim_active=0 for “train off,” not a fault (of ~51 monitored DOSTO CCUs only ~8 were online at check time), and Zabbix cannot show that a blade was physically swapped. The remaining-fleet count and completion target must therefore come from the SPL/Nomad blade-replacement work log, not from monitoring. Internal only.

## Media Content

Not applicable. No portal media content is delivered by Nomad as part of this release.

## Testing and Validation

The onboard-network baseline is validated per train through a Layer-2 network health check (switch error counters, RSTP topology, trunk states, and end-to-end vlan7 reachability to the Stadler firewall) and, where applicable, throughput sampling. Results are recorded per train and summarised in the customer network health-check reports. Software validation is covered by the Nomad Connect DEVOPS Test Exit Report referenced in Section 1.