DOSTO NEU Train Monitoring

Alarm Catalogue

Nomad Digital | OBB DOSTO NEU Fleet

June 2026

Document Version: v1.0

CONFIDENTIAL

# 1. Introduction

Nomad Digital operates a Zabbix-based network monitoring platform for the OBB DOSTO NEU fleet. This platform continuously monitors the health and availability of all onboard network equipment, providing real-time alerting to the Nomad Digital Network Operations Centre (NOC).

This document provides a complete catalogue of all monitoring alarms configured for the DOSTO NEU fleet. For each alarm, the catalogue describes the condition that triggers it, the operational impact, and the recommended response action.

## 1.1 Monitored Device Types

Alarms are configured for the following device types on each trainset:

| Device Type | Description |
| --- | --- |
| CCU (Onboard Computer) | The Nomad Digital Central Control Unit running on each trainset. Manages Wi-Fi, portal, modems, GPS, and all onboard services. |
| Access Points (APs) | Westermo RT610LV wireless access points providing passenger Wi-Fi coverage throughout the train. |
| VDS Rail Consist Switches | Stadler-supplied managed Ethernet switches forming the onboard network fabric between coaches. |
| Modems | Cellular modems providing the bonded 4G/5G uplink for onboard internet connectivity. |
| Stadler Subsystems | Third-party systems monitored via the CCU: Video Surveillance System (VSS) cameras and recorder, and Stadler Display Equipment (SDE). |

## 1.2 Severity Levels

Each alarm is assigned one of four severity levels:

| Severity | Description | Alarm Count |
| --- | --- | --- |
| Disaster | Critical failure causing complete loss of service. Immediate action required. | 0 |
| High | Significant fault affecting passenger-facing or operational services. Urgent investigation required. | 29 |
| Warning | Degraded condition that may impact service if not addressed. Action required within business hours. | 28 |
| Information | Advisory notification. No immediate action required; monitor for trends. | 3 |
| TOTAL |  | 60 |

# 2. CCU (Onboard Computer) Alarms

The CCU is the central onboard Linux computer managing all Nomad Digital services on the train. The following 35 alarms are configured for CCU monitoring.

| Alarm Name | Severity | Condition / What Triggers | Impact | Recommended Action |
| --- | --- | --- | --- | --- |
| No connectivity — MAR3 not active | High | The CCU's MAR3 cellular bonding client has stopped reporting statistics. | Loss of onboard internet connectivity for passengers and remote monitoring. | Check CCU status remotely. Restart MAR3 service or reboot CCU if unresponsive. |
| MAR3 unavailable by ICMP | High | The CCU cannot be reached via ICMP ping. | CCU is offline or unreachable. All onboard services may be affected. | Investigate network path to CCU. Check train power and physical connections. |
| Engineering Pages not running | High | The engineering_pages.service systemd unit is not active. | Engineering diagnostic interface unavailable. | Run systemctl restart engineering_pages.service. If unresolved, run nd-systemupdate.sh up or reboot. |
| CCU not listening on engineering port | High | The CCU web server is not accepting connections on the configured engineering port. | Engineering diagnostics interface inaccessible. | Check engineering_pages.service status and port configuration. |
| Auth-Server not running | High | The onboard authentication server process is not active. | Passengers may be unable to authenticate to the portal or Wi-Fi network. | Restart auth-server service on the CCU. |
| DHCP not active | High | The onboard DHCP service has stopped while the train is in service. | New passenger devices cannot obtain an IP address and cannot connect to the Wi-Fi portal. | Restart DHCP service on CCU. Investigate root cause. |
| GPS web service not active | High | The GPS web service daemon is not running. | Train position data unavailable for operational and passenger information systems. | Restart GPS service. Check GPS hardware if service fails to start. |
| No external ID configured | High | The CCU has no external train ID set in its configuration. | Train cannot be identified by back-office systems. Reporting and telemetry may fail. | Set the correct external ID via the provisioning system. |
| Telemetry application failed | High | The telemetry service used by the Asset Register and Mobile Application is not running. | Asset tracking and mobile application data feeds are unavailable. | Restart telemetry service. Escalate to Nomad Digital if persistent. |
| Docker Portal not responsive | High | The Docker container hosting the passenger portal is not responding to internal ping. | Passenger-facing portal is likely unavailable. | Run docker ps on CCU. Restart portal container if stopped. |
| Docker Portal IP mismatch | High | The Docker portal container's IP address does not match the expected value. | Portal may be unreachable from passenger devices. | Investigate Docker network configuration. Restart container or Docker daemon. |
| Docker portal not healthy | High | Docker health check reports the portal container is not in a healthy or starting state. | Portal service degraded or unavailable. | Check container logs with docker logs. Restart container if unhealthy. |
| Docker not in running state | High | The Docker service itself is not running on the CCU. | All containerised services (portal, etc.) are offline. | Start Docker service: systemctl start docker. |
| Portal showing 404 | High | HTTP request to the passenger portal returns a 404 error. | Passengers cannot access the Wi-Fi portal. | Check portal container and web server configuration. |
| Portal Connecttoweb page showing error | High | The portal's internet redirect page is returning an error. | Passengers connected to Wi-Fi cannot be redirected to the internet. | Check portal application logs and configuration. |
| Landing page certificate expired | High | The TLS certificate for the passenger portal landing page has expired. | Passengers will see browser security warnings and may be unable to connect. | Renew and deploy updated TLS certificate immediately. |
| Engineering collector not running | Warning | The engineering_collector.service systemd unit is not active. | Engineering data collection paused; diagnostic data may be missing. | Run systemctl restart engineering_collector.service. |
| Engineering server not running | Warning | The engineering_server.service systemd unit is not active. | Engineering server interface unavailable. | Run systemctl restart engineering_server.service. |
| Engineering page cannot be curled | Warning | Automated curl check to the engineering web page failed. | Engineering interface may be unresponsive. | Check engineering_pages.service and web server on port 8005. |
| No GPS for 10 minutes | Warning | The GPS chip or daemon has not provided any position data for 10 minutes. | Train position unknown. May affect passenger information systems. | Check GPS hardware and daemon. Investigate if persists after tunnel exits. |
| Invalid GPS for 90 minutes | Warning | GPS has not provided a valid fix for 90 minutes (possible extended tunnel or cover). | Position data unreliable for extended period. | Monitor; expected in long tunnels. Escalate if occurring in open areas. |
| Certificate expiring in 7 days | Warning | The TLS certificate for the passenger portal will expire within 7 days. | Portal will become inaccessible to passengers after expiry. | Renew and deploy certificate before expiry. |
| Portal HTTP request failed | Warning | Automated HTTP health check to the portal returned a non-200 status code. | Portal may be partially unavailable. | Investigate portal application and web server. |
| Sync request failed (>3 occasions) | Warning | Data synchronisation between CCU and back-office has failed more than 3 times. | Reporting data may be stale or missing. | Check network connectivity and back-office sync service. |
| Portal user experience degraded | Warning | Portal response time exceeds 1000 ms. | Passengers experience slow portal load times. | Investigate CCU load and portal application performance. |
| High load average (>0.7) | Warning | CCU 5-minute load average exceeds 0.7. | CCU may be under resource pressure, potentially affecting all onboard services. | Investigate running processes. Restart non-essential services if needed. |
| Free memory less than 2 GB | Warning | Available system memory has dropped below 2 GB. | Risk of service degradation or out-of-memory kills. | Investigate memory usage. Restart services if needed. |
| High CPU utilisation | Warning | CPU utilisation exceeds the configured threshold. | CCU performance degraded. | Identify and investigate high-CPU processes. |
| Low disk space on /var/log | Warning | Free space on /var/log partition is below 20%. | Log writes may fail; potential for service instability. | Rotate or archive logs. Investigate what is filling the partition. |
| NTP not active | Warning | The NTP daemon is not running. | Onboard system clock may drift, affecting logs, certificates, and time-sensitive services. | Restart NTP service. |
| Puppet environment not production | Warning | The CCU's Puppet environment is not set to production. | CCU may not receive standard configuration updates. | Set Puppet environment to production via management system. |
| Host restarted recently | Warning | CCU uptime is less than 10 minutes. | Advisory — CCU has rebooted. Services may still be starting up. | Monitor to confirm all services recover. Investigate unexpected reboots. |
| Flash erase count >10,000 | Information | The average NAND flash erase count on the CCU exceeds 10,000 cycles. | Flash storage approaching end-of-life threshold. | Flag for hardware replacement planning. |
| GPS Data from Blade 1/2 invalid | Information | GPS data from one of the redundant GPS blades is invalid. | Redundancy reduced; single GPS source in use. | Monitor. Investigate blade health if both blades report invalid simultaneously. |
| Engineering Pages: credential check failed | Information | Automated login check to Engineering Pages failed authentication. | Automated monitoring of engineering pages may be impaired. | Verify credentials configured for the engineering pages health check. |

# 3. Modem Alarms

Each DOSTO NEU trainset is equipped with multiple cellular modems providing bonded 4G/5G uplink connectivity. The following 6 alarms monitor modem and SIM health.

| Alarm Name | Severity | Condition / What Triggers | Impact | Recommended Action |
| --- | --- | --- | --- | --- |
| SIM 0–5 locked (invalid PIN) | High | One or more SIM cards are locked due to an invalid PIN code. | Affected modem cannot connect to the cellular network. Redundancy reduced or lost. | Unlock SIM via carrier or replace SIM. Check PIN configuration. |
| VLAN 21–26 state down | High | One or more modem VLANs (21–26) are reporting a down state. | The cellular bonding path associated with that VLAN is unavailable. | Check modem and VLAN configuration on CCU. |
| Modem 0–5 not active | Warning | A modem is not in an active/connected state. | Cellular redundancy reduced. Connectivity may be degraded if multiple modems affected. | Check modem hardware and SIM. Restart modem if needed. |
| Modem 0–5 missing SIM | Warning | A SIM card is not detected in the modem slot. | That modem cannot connect to cellular networks. | Follow SIM replacement procedure (Confluence). Report to Nomad Digital. |
| Modem stuck in bootloader | Warning | A modem is stuck in bootloader mode and cannot initialise. | Modem non-functional. Cellular redundancy reduced. | Follow modem recovery procedure (Confluence). Replace if unrecoverable. |
| Modem offline for 24 hours | Warning | A modem has been continuously offline for 24 hours. | Extended loss of cellular path. Overall connectivity may be significantly impacted. | Investigate modem hardware. Escalate to Nomad Digital if unresolved. |

# 4. Access Point (Wi-Fi) Alarms

Westermo RT610LV access points provide passenger Wi-Fi coverage throughout each trainset. The following 9 alarms monitor AP availability, configuration, firmware, and performance.

| Alarm Name | Severity | Condition / What Triggers | Impact | Recommended Action |
| --- | --- | --- | --- | --- |
| Access point unreachable by ICMP | High | The CCU cannot ping the access point. | Wi-Fi service in the affected coach/zone is likely down. | Check AP power and network connection. Reboot AP if reachable via management. |
| Access point config version mismatch | High | The AP's running configuration does not match the expected version. | AP may be running an incorrect configuration, potentially affecting Wi-Fi service. | Push correct configuration to AP via OBN management system. |
| Access point firmware version mismatch | High | The AP is not running the target firmware version. | AP may lack bug fixes or feature updates required for correct operation. | Push target firmware to AP via OBN management system. |
| Access point high CPU usage (>90%) | High | AP CPU utilisation has exceeded 90% over the last 5 minutes. | AP performance degraded; may cause Wi-Fi instability for passengers. | Investigate traffic load. Reboot AP if unresponsive. |
| Access point high memory usage (>90%) | High | AP memory usage has exceeded 90%. | Risk of AP crash or service degradation. | Reboot AP. Investigate if recurring. |
| Access point SNMP trap received | High | The AP has sent an SNMP trap, indicating an internal event or fault. | May indicate hardware fault or service disruption. | Review trap details. Investigate AP logs. |
| Access point 0 connected users (total) | High | The AP reports zero connected passenger devices. | AP may have stopped serving Wi-Fi, even if reachable. | Check AP radio status. Reboot if radios appear down. |
| Access point 0 users on individual radio | High | A specific radio on the AP (radio 0, 0_1, 0_2, 1, 1_1, or 1_2) reports zero connected users. | One frequency band or sector is not serving passengers. | Check radio configuration and status on the AP. |
| Access point reboot detected | Warning | AP uptime counter has reset within the last 5 minutes, indicating a reboot. | Brief Wi-Fi interruption in the affected area. | Monitor for repeated reboots. Investigate cause if frequent. |

# 5. VDS Rail Consist Switch Alarms

VDS Rail Consist Switches form the onboard Ethernet fabric interconnecting coaches. The following 7 alarms monitor switch availability, configuration, firmware, and inter-coach link health.

| Alarm Name | Severity | Condition / What Triggers | Impact | Recommended Action |
| --- | --- | --- | --- | --- |
| Switch unavailable by ICMP | High | The CCU cannot ping the consist switch. | The switch and all devices connected to it may be unreachable. | Check switch power and inter-coach trunk connections. Investigate physical layer. |
| DTL link failed (ports 1+3 or 2+4) | High | Both ports forming a Dual Trunk Link (DTL) are simultaneously down. | Complete loss of a redundant inter-coach trunk pair. Network segment may be isolated. | Investigate inter-coach cable connections. Escalate to Stadler if physical fault suspected. |
| Potential split-brain issue | High | The number of active uplinks has dropped below 2, indicating possible network partitioning. | Part of the consist network may be isolated from the rest. | Investigate inter-coach trunk status across the consist. Check for cable faults. |
| Switch config version mismatch | Warning | The switch's running configuration does not match the expected version. | Switch may be running an incorrect configuration. | Push correct configuration via OBN management system. |
| Switch firmware version mismatch | Warning | The switch is not running the target firmware version. | Switch may lack required updates. | Push target firmware via OBN management system. |
| Port 1/2/4/5/6/10 operational status down | Warning | A configured switch port has gone operationally down (link lost). | The device connected to that port is unreachable. May affect coaches or devices depending on topology. | Check physical connection and device at the far end of the link. |
| Link Aggregation 1/2 operational status down | Warning | A link aggregation group (LAG) on the switch is down. | Inter-coach trunk redundancy lost or inter-coach connectivity affected. | Investigate both member ports of the LAG. Check inter-coach cable connections. |

# 6. Stadler Subsystem Alarms

Nomad Digital monitors selected Stadler subsystems that are visible via the CCU. Note: Nomad Digital monitors and reports on these alarms; resolution of hardware faults lies with Stadler and OBB. The following 3 alarms are configured.

| Alarm Name | Severity | Condition / What Triggers | Impact | Recommended Action |
| --- | --- | --- | --- | --- |
| SDE 1 reporting down | Warning | The Stadler Display Equipment (SDE) unit 1 is reporting a down state via the CCU. | Passenger information displays in the affected coach may be non-functional. | Raise with OBB operations. Stadler is responsible for SDE hardware. |
| VSS-CAM 1–5 reporting down | Warning | One or more Video Surveillance System (VSS) cameras are reporting down. | CCTV coverage in the affected area is lost. | Raise with OBB operations. Stadler is responsible for VSS hardware. |
| VSS-REC reporting down | Warning | The VSS recording unit is reporting down. | Onboard CCTV recording is not functioning. | Raise with OBB operations. Stadler is responsible for VSS hardware. |

# 7. Contact Information

For queries relating to this alarm catalogue or the DOSTO NEU monitoring platform, please contact:

| Organisation | Nomad Digital |
| --- | --- |
| Team | Network Operations Centre (NOC) |
| Email | noc@nomadrail.com |
| Document Owner | Nomad Digital Engineering |
| Document Version | v1.0 — June 2026 |