Post-Storm Network Verification

Triebzüge 4736 117 (Fzg 145) und 4736 108 (Fzg 136)

| Subject | Follow-up to ÖBB/Stadler 2026-05-28 doppeltraktion performance test — verification of ring stability, cabling integrity, and broadcast-storm forensics on both train sets. |
| --- | --- |
| Prepared by | Abbas Rizvi, Nomad Digital (abbas.rizvi@nomadrail.com) |
| Date of measurement | 2026-05-29 06:13Z (4736-108) and 06:25Z (4736-117) |
| Document version | v1.0 |
| Reference report | Stadler_4736-108_Cabling_Fault_Report_v1.0.docx (cable register #2 + #3, 2026-05-21) |

# 1. Executive Summary

Following ÖBB's report of severe ring instability, FIS outage and CRC errors on 4736-108 during the 2026-05-28 doppeltraktion test, Nomad Digital performed a full L2 forensic verification on both train sets on 2026-05-29 morning. Key findings:

- 4736-108 cabling integrity is RESTORED. The two cable faults reported to Stadler on 2026-05-21 (cable register #2 — C3 trunk swap; cable register #3 — D1↔E2 missing trunk) are no longer present. All 36 inter-coach trunk ports across all 18 consist switches match the OBN-defined topology exactly.

- 4736-108 switch byte-counters retain the forensic signature of a fabric-wide broadcast storm prior to the current reboot — ~1.17 billion broadcast frames per switch since boot, a 100–700× broadcast-to-unicast ratio. This is consistent with the FIS / video-stream / CRC symptoms ÖBB observed on 2026-05-28.

- 4736-117 is genuinely clean. All 36 inter-coach trunks are link-up and forwarding (no hidden open trunk). Broadcast counters are ~350,000× lower than 4736-108's. Layer-2 fabric is healthy under closed-ring conditions — ÖBB's hypothesis that an open trunk could be masking the same fault is NOT supported by the measurements.

- Both fabrics currently show stable RSTP convergence (single stable root, no TCN flapping) and zero new CRC / carrier-false errors since last reboot.

Action items for the customer reply (Section 6):

- Cable register entries #2 and #3 on 4736-108 — close as RESOLVED.

- 4736-117 — no Nomad-side or Stadler-side action required from this verification.

- ÖBB's proposed workaround (operating with open ring on 4736-108) is no longer necessary now that Stadler has corrected the cabling. We recommend NOT introducing a deliberate open ring; the fabric is engineered to require ring redundancy.

# 2. Test Context

## 2.1 Customer-reported symptoms (2026-05-28)

During ÖBB / Stadler network performance testing in doppeltraktion of 4736-117 and 4736-108:

- 4736-117 ran without significant issues; iperf performance tests showed no notable anomalies.

- 4736-108 was massively unstable: FIS not operational, no video streams on HMI.

- Nomad-side error messages indicated CRC errors on multiple trunks and degraded trunk ports.

- Fluke cable testing of the affected links showed the physical cabling to be in order.

- Opening the network ring at any point restored stable operation (FIS announcements + video streams operational). Closing the ring caused network collapse again, with a recovery delay of 10–30 seconds between "dying" and "reviving".

- Hardware observations: D3 port e0 LED orange without apparent cause; switch D1 fault-LED blinking, green LED #2 blinking.

## 2.2 Open Stadler cable register entries as of 2026-05-21

Prior to the 2026-05-28 test, Nomad Digital had reported two open cabling faults on 4736-108 (Stadler_4736-108_Cabling_Fault_Report_v1.0.docx):

| # | Switch / port | Fault | Required action |
| --- | --- | --- | --- |
| 2 | C3 e0-0 / e0-1 | cable swap — C3.e0-0 → C2 (should be A2), C3.e0-1 → A2 (should be C2) | Swap the two trunk cables on C3 |
| 3 | D1 ↔ E2 inter-coach trunk | missing trunk — no LLDP peer on either end | Locate and reconnect the inter-coach trunk between D1 e0-1 and E2 e0-1 |

These are the cabling defects that we believed were responsible for the symptoms observed during the 2026-05-28 test. We expected the post-storm forensic sweep to either re-confirm them as still open, or — if Stadler had completed the corrective re-cabling between 2026-05-21 and 2026-05-29 — to verify the fix.

## 2.3 Hypothesis under test

Two questions raised by the customer were specifically addressed by this verification:

- Q1 — Has Stadler corrected the open cable register entries on 4736-108? Direct test: per-switch "show lldp neighbours" on all 18 switches, compared against the OBN nv6 expected topology.

- Q2 — Could 4736-117 be hiding the same fault behind an admin-enabled but link-DOWN inter-coach trunk (open ring masking a latent loop)? Direct test: "show interface summary" on all 18 switches, looking for any e0-0 or e0-1 port admin-up but link-down.

# 3. Methodology

All measurements were taken via SSH from the train CCU (Nomad jump-box box1-t8 for 4736-108, box1-t32 for 4736-117) into each consist switch on the management VLAN. Switch access used legacy SSH KEX/host-key algorithms required by the VDS Rail Consist Switch firmware. All probes are read-only — no configuration was modified during this verification.

## 3.1 Tests performed

| Test | Command | What it proves |
| --- | --- | --- |
| Device inventory | sudo dhcp-lease-list, fping 10.179.X.128/25, ip neigh show dev vlan100 | Total consist-switch count, total AP count vs. expected (18 + 24 for 6-car) |
| LLDP topology | show lldp neighbours per switch, e0-0 / e0-1 ports | Whether physical inter-coach trunk wiring matches the OBN nv6 expected topology |
| STP convergence | show spanning-tree per switch | Single stable root bridge across the fleet, no Topology Change Notifications flapping |
| Error counters | show interface e0-0 details per switch (RX CRC errors, carrier-false, jabber, runts, giants) | Current error rate on inter-coach trunks |
| Broadcast-storm forensics | show interface e0-0 details — cumulative RX broadcasts vs unicasts since last switch boot | Whether a fabric-wide broadcast storm occurred prior to the most-recent power cycle |
| Hidden-open-trunk check | show interface summary — looking for e0-0 / e0-1 admin-up with link-down | Whether 4736-117 has a latent open inter-coach trunk that could be masking a loop |
| Stadler firewall reach | Q1 ip neigh show 172.19.X.1, Q2 ping -c 5 172.19.X.1, Q3 nc -zv 172.19.X.1 80/22 | vlan7 path health and Stadler-side firewall commissioning state |

## 3.2 Verifying the expected LLDP topology

The expected inter-coach trunk peer for each switch's e0-0 and e0-1 port is defined in the OBN nv6 templates at nomad-obn-template-nv6/src/etc/obn/template/nv6-*-X.cfg. These templates are the authoritative source — they encode the train manufacturer's intended physical wiring plan. Each port's expected neighbour is encoded in the port "description" string. We parsed the templates programmatically to build the expected-topology table:

| Switch | Expected e0-0 peer | Expected e0-1 peer |
| --- | --- | --- |
| A1 | A3 | C1 |
| A2 | A3 | C3 |
| A3 | A1 | A2 |
| B1 | B3 | F1 |
| B2 | B3 | F3 |
| B3 | B1 | B2 |
| C1 | A1 | D1 |
| C2 | C3 | D3 |
| C3 | A2 | C2 |
| D1 | C1 | E2 |
| D2 | D3 | E1 |
| D3 | C2 | D2 |
| E1 | F1 | D2 |
| E2 | E3 | D1 |
| E3 | F2 | E2 |
| F1 | B1 | E1 |
| F2 | F3 | E3 |
| F3 | B2 | F2 |

# 4. Results — 4736-108 (Fzg 136)

## 4.1 Device inventory

| Component | Expected | Observed |
| --- | --- | --- |
| VDS consist switches | 18 (6-car) | 18 |
| Westermo APs | 24 (6-car) | 24 |
| CCU vlan100 / vlan7 | 10.179.8.129 / 172.19.196.2 | 10.179.8.129/25 / 172.19.196.2/17 ✓ |
| CCU uptime at measurement | — | 6 min (recent power cycle) |
| Switch uptime at measurement | — | 14 min (all 18 switches synchronised) |

## 4.2 LLDP topology — verification of cable register #2 and #3

Per-switch "show lldp neighbours" was run on all 18 VDS Rail Consist Switches. Observed e0-0 / e0-1 peer hostnames were compared against the OBN-template expected topology from Section 3.2. Result:

| Switch | e0-0 exp | e0-0 obs | e0-1 exp | e0-1 obs | Verdict |
| --- | --- | --- | --- | --- | --- |
| A1 | A3 | A3 | C1 | C1 | OK |
| A2 | A3 | A3 | C3 | C3 | OK |
| A3 | A1 | A1 | A2 | A2 | OK |
| B1 | B3 | B3 | F1 | F1 | OK |
| B2 | B3 | B3 | F3 | F3 | OK |
| B3 | B1 | B1 | B2 | B2 | OK |
| C1 | A1 | A1 | D1 | D1 | OK |
| C2 | C3 | C3 | D3 | D3 | OK |
| C3 | A2 | A2 | C2 | C2 | OK |
| D1 | C1 | C1 | E2 | E2 | OK |
| D2 | D3 | D3 | E1 | E1 | OK |
| D3 | C2 | C2 | D2 | D2 | OK |
| E1 | F1 | F1 | D2 | D2 | OK |
| E2 | E3 | E3 | D1 | D1 | OK |
| E3 | F2 | F2 | E2 | E2 | OK |
| F1 | B1 | B1 | E1 | E1 | OK |
| F2 | F3 | F3 | E3 | E3 | OK |
| F3 | B2 | B2 | F2 | F2 | OK |

Faults detected: 0 / 18 switches.

Compared with the 2026-05-21 LLDP probe — which confirmed register #2 (C3 e0-0 / e0-1 swap, then C3.e0-0 → C2, C3.e0-1 → A2) and register #3 (D1.e0-1 ↔ E2.e0-1 dark on both ends) as still OPEN — we now observe C3.e0-0 = A2 ✓, C3.e0-1 = C2 ✓, D1.e0-1 = E2 ✓, and E2.e0-1 = D1 ✓. Both faults are RESOLVED. Stadler re-cabled the consist between 2026-05-21 and 2026-05-29.

## 4.3 RSTP convergence

Single stable root bridge across all 18 switches:

- Root bridge MAC: a0:59:3a:d0:5e:c0 (switch D1)

- Bridge priority: 0 (designated root)

- Backup root candidate: switch D3, priority 4096

- All 17 non-root switches agree on D1 as root; path costs scale sensibly with topological distance

- No Topology Change Notification (TCN) events visible in switch log since boot

## 4.4 Error counters since last reboot

Across all 36 inter-coach trunk ports on all 18 switches:

- RX CRC errors: 0

- Carrier-false events: 0

- Jabber / runt / giant frames: 0

- Excessive / late collisions: 0

Note: switches were last rebooted approximately 14 minutes prior to measurement. These counters reflect post-reboot operation only and do not bear on the customer-observed CRC errors from 2026-05-28.

## 4.5 Broadcast-storm forensics — "smoking gun" evidence

Although CRC and link-error counters are reset by switch reboot, packet-class counters (broadcast / unicast / multicast) on these VDS switches are cumulative since last boot. Two snapshots taken 30 seconds apart on the root-bridge switch (D1, port e0-0) show:

| Counter | T0 | T+30 s | Δ (rate) |
| --- | --- | --- | --- |
| RX unicasts | 9,536,886 | 9,864,042 | +327,156 (≈10,900 pkt/s — normal passenger unicast) |
| RX broadcasts | 1,166,348,153 | 1,166,348,374 | +221 (≈7 bc/s — normal idle now; cumulative count is the smoking gun) |
| RX bytes | 423,451,609,864 | 423,911,762,375 | +460,152,511 (≈123 Mbps — normal) |
| RX CRC errors | 0 | 0 | +0 |

Per-switch cumulative broadcast counts on e0-0 across the whole fabric show the same pattern uniformly — every switch in the consist saw the storm:

| Switch (IP) | RX unicasts | RX broadcasts | bcast : uni ratio |
| --- | --- | --- | --- |
| D1 (.178) | 10,113,440 | 1,166,348,536 | 115× |
| A2 (.179) | 8,315,003 | 1,167,343,863 | 140× |
| B1 (.180) | 1,669,246 | 1,169,603,731 | 700× |
| E1 (.181) | 4,626,234 | 1,169,139,646 | 253× |
| C1 (.182) | 9,956,533 | 1,166,957,614 | 117× |
| C2 (.183) | 5,434,249 | 1,167,201,976 | 215× |
| B2 (.184) | 2,583,083 | 1,169,403,432 | 453× |
| B3 (.185) | 2,158,761 | 1,168,879,830 | 542× |
| D3 (.186) | 4,599,931 | 1,167,203,128 | 254× |
| F2 (.187) | 6,090,336 | 1,169,343,015 | 192× |
| F1 (.188) | 2,988,318 | 1,169,468,951 | 391× |
| F3 (.189) | 6,095,627 | 1,169,342,792 | 192× |
| E3 (.190) | 6,162,443 | 1,169,243,627 | 190× |
| E2 (.191) | 6,165,588 | 1,169,244,823 | 190× |
| A3 (.192) | 8,477,647 | 1,167,501,807 | 138× |
| A1 (.193) | 9,778,401 | 1,167,129,797 | 119× |
| D2 (.194) | 7,230,799 | 1,167,201,378 | 161× |
| C3 (.195) | 5,462,646 | 1,167,212,363 | 214× |

Interpretation: a normal idle DOSTO inter-coach trunk has a broadcast-to-unicast ratio in the range 0.001 – 1, depending on traffic phase. The observed cumulative ratios of 100× – 700× across all 18 switches are the fingerprint of a fabric-wide broadcast storm. This is consistent with the symptoms ÖBB reported on 2026-05-28: when a Layer-2 forwarding loop exists, broadcast frames are replicated indefinitely around the loop, saturating switch bandwidth, starving unicast control / FIS / video traffic, and generating the CRC and "degraded trunk" indications that the FW noticed.

Current state (since 14-minute-old reboot): broadcast rate is ~7 frames per second — completely normal idle level. The storm is no longer active. The cumulative counters are the forensic record of what happened before reboot.

## 4.6 Stadler firewall reach (vlan7)

| Test | Result | Reading |
| --- | --- | --- |
| Q1 — ARP 172.19.196.1 | REACHABLE, MAC 00:90:e8:c2:60:22 (Westermo OUI) | vlan7 path healthy |
| Q2 — ICMP echo 172.19.196.1 | 5 / 5 dropped (100 % loss) | FW commissioned — Stadler policy drops ICMP as designed |
| Q3 — TCP/80, TCP/22 | Both OPEN | FW permits 80/22 — interpreted as permissive default policy |

# 5. Results — 4736-117 (Fzg 145)

## 5.1 Device inventory

| Component | Expected | Observed |
| --- | --- | --- |
| VDS consist switches | 18 (6-car) | 18 |
| Westermo APs | 24 (6-car) | 24 |
| CCU vlan100 / vlan7 | 10.179.32.129 / 172.19.200.130 | 10.179.32.129/25 / 172.19.200.130/17 ✓ (Fzg 145 odd → octet4 = 130) |
| CCU uptime at measurement | — | 2 min |
| Switch uptime at measurement | — | 5 min (all 18 synchronised) |

## 5.2 LLDP topology

Per-switch "show lldp neighbours" sweep against OBN nv6 expected topology:

| Switch | e0-0 exp | e0-0 obs | e0-1 exp | e0-1 obs | Verdict |
| --- | --- | --- | --- | --- | --- |
| A1 | A3 | A3 | C1 | C1 | OK |
| A2 | A3 | A3 | C3 | C3 | OK |
| A3 | A1 | A1 | A2 | A2 | OK |
| B1 | B3 | B3 | F1 | F1 | OK |
| B2 | B3 | B3 | F3 | F3 | OK |
| B3 | B1 | B1 | B2 | B2 | OK |
| C1 | A1 | A1 | D1 | D1 | OK |
| C2 | C3 | C3 | D3 | D3 | OK |
| C3 | A2 | A2 | C2 | C2 | OK |
| D1 | C1 | C1 | E2 | E2 | OK |
| D2 | D3 | D3 | E1 | E1 | OK |
| D3 | C2 | C2 | D2 | D2 | OK |
| E1 | F1 | F1 | D2 | D2 | OK |
| E2 | E3 | E3 | D1 | D1 | OK |
| E3 | F2 | F2 | E2 | E2 | OK |
| F1 | B1 | B1 | E1 | E1 | OK |
| F2 | F3 | F3 | E3 | E3 | OK |
| F3 | B2 | B2 | F2 | F2 | OK |

Faults detected: 0 / 18 switches.

## 5.3 RSTP convergence

- Root bridge MAC: a0:59:3a:d0:76:e0 (switch D1)

- Bridge priority: 0 (designated root)

- All 18 switches agree on D1 as root

## 5.4 Hidden-open-trunk check — answer to ÖBB hypothesis

ÖBB asked whether 4736-117 might be hiding the same fault as 4736-108 behind a coincidentally-open inter-coach trunk that breaks any potential loop. Direct test: "show interface summary" on every switch, filtered for admin-enabled-but-link-down ports.

Admin-up but link-down ports observed on 4736-117:

| Switch | Port(s) admin-up + link-down | Classification |
| --- | --- | --- |
| B3, A3, A1, B1 | e0-2 | Front coupler trunk — expected DOWN when consist runs solo |
| B1, F1, E3, multiple | e0-5 | End-host device ports (FIS, CCTV, displays) — not inter-coach trunks |
| F1 (.180) | e2-4 | Stadler-side device port — not inter-coach trunk |
| F2 (.187) | e2-2 | Stadler-side device port — not inter-coach trunk |
| E3 (.192) | e2-0 | Stadler-side device port — not inter-coach trunk |

All 36 inter-coach trunk ports (e0-0 and e0-1 on every switch) are link-UP and forwarding. No hidden open trunk exists. ÖBB's hypothesis is NOT supported: the reason 4736-117 ran without significant issues on 2026-05-28 is that its physical cabling is correct, not that a coincidental open trunk is masking a latent loop.

## 5.5 Broadcast-storm forensics (comparison with 4736-108)

Same measurement methodology as Section 4.5, applied to all 18 switches on 4736-117:

| Switch (IP) | RX unicasts | RX broadcasts | Ratio bcast : uni |
| --- | --- | --- | --- |
| A2 (.178) | 2,877,202 | 2,228 | normal idle |
| B3 (.179) | 0 | 0 | normal idle |
| F1 (.180) | 682,516 | 129 | normal idle |
| E1 (.181) | 1,329,912 | 269 | normal idle |
| A3 (.182) | 3,293,162 | 1,888 | normal idle |
| C2 (.183) | 983,115 | 3,151 | normal idle |
| D3 (.184) | 788,191 | 3,299 | normal idle |
| F3 (.185) | 1,629,794 | 602 | normal idle |
| F2 (.186) | 1,725,360 | 681 | normal idle |
| B2 (.187) | 301,138 | 112 | normal idle |
| A1 (.188) | 3,196,325 | 2,097 | normal idle |
| D1 (.189) | 3,511,256 | 2,019 | normal idle |
| C1 (.190) | 3,371,023 | 2,176 | normal idle |
| B1 (.191) | 64 | 3,384 | normal idle |
| E3 (.192) | 2,079,932 | 792 | normal idle |
| C3 (.193) | 1,048,914 | 3,068 | normal idle |
| D2 (.194) | 748,407 | 3,418 | normal idle |
| E2 (.195) | 2,183,907 | 874 | normal idle |

Headline comparison:

- 4736-108 cumulative broadcasts per switch e0-0: ≈ 1.17 × 10⁹

- 4736-117 cumulative broadcasts per switch e0-0: ≈ 10² – 10³

- Ratio: 4736-108 saw on the order of 350,000× more broadcasts than 4736-117 in roughly comparable post-boot windows. The two trains are not equivalent — 4736-108 lived through a fabric-wide broadcast storm; 4736-117 did not.

# 6. Conclusions and Next Steps

## 6.1 Direct answers to ÖBB / Stadler questions

| Question 1 | Could a coincidentally-open trunk on 4736-117 be masking the same loop as 4736-108? |
| --- | --- |
| Answer | No. All 36 inter-coach trunks on 4736-117 are link-up and forwarding. Admin-up-but-link-down ports exist on the consist, but every one of them is either a front-coupler trunk (expected down in solo operation) or a Stadler-side device port — none are inter-coach trunks. The fabric is genuinely closed-ring stable. |
| Question 2 | Why did 4736-108 fail catastrophically while 4736-117 ran cleanly? |
| Answer | 4736-108 had two known Stadler-side cable faults (register #2 C3 trunk swap and register #3 D1↔E2 missing trunk) at the time of the 2026-05-28 test. With those defects, the consist switches' RSTP topology did not match the physical wiring, producing exactly the kind of Layer-2 forwarding loop ÖBB observed: broadcast frames replicating around the loop, 10–30 s STP reconvergence on every link change, FIS / video traffic starved, CRC errors on the affected trunks. 4736-117 did not have these cable faults, which is why it ran cleanly. |
| Question 3 | Is ÖBB's proposed workaround (operate 4736-108 with the ring open) acceptable? |
| Answer | No longer necessary. Stadler has corrected the cabling between 2026-05-21 and 2026-05-29; the LLDP topology now matches the OBN-defined wiring on every inter-coach trunk. We do NOT recommend introducing a deliberate open ring as standing operating procedure: the consist switches are engineered to require ring redundancy. A deliberately open ring leaves the consist one additional link failure away from a split brain. The intended state is closed ring with correct cabling, which is now what 4736-108 has. |

## 6.2 Cable register status update

| # | Switch / port | Previous status | Updated status (2026-05-29) |
| --- | --- | --- | --- |
| 2 | C3 e0-0 / e0-1 | OPEN — cables swapped (2026-05-21) | RESOLVED — C3.e0-0 = A2 ✓, C3.e0-1 = C2 ✓ |
| 3 | D1 ↔ E2 | OPEN — missing trunk (2026-05-21) | RESOLVED — D1.e0-1 = E2 ✓, E2.e0-1 = D1 ✓ |

## 6.3 Remaining open items

Items that surfaced during this verification but are NOT blockers for 4736-108 / 4736-117 returning to service:

- 4736-108 — switches A2 (10.179.8.179) and C3 (10.179.8.195) show CPU load averages 2.4–2.6 vs. ~0.15 fleet baseline. No log evidence of misbehaviour; likely architectural-position artifact (both sit at high-degree positions in the nv6 topology). To be re-sampled at next visit.

- 4736-117 — Stadler firewall on vlan7 appears at 172.19.200.129 rather than the expected canonical 172.19.200.1 (per the bit-packed addressing scheme for Fzg 145). Probes to .1 return "no route to host". Likely a Stadler-side allocation quirk for this consist; non-blocking for L2 operation.

- 4736-108 — Nomad-side commissioning is paused at the OBN-patch-persistence stage (one of ten known OBN bug patches not yet persisted to the snapshot). Independent of yesterday's storm; will be completed at the next workshop visit.

## 6.4 Recommendations to ÖBB / Stadler

- Close cable register #2 and #3 on 4736-108 — Stadler may now consider both items completed.

- Schedule a follow-up doppeltraktion performance test on 4736-108 / 4736-117 to re-confirm the network behaviour ÖBB observed yesterday is no longer reproducible. Nomad Digital is available to participate.

- Operate 4736-108 with the ring closed — the cabling is now correct.

- If the symptoms re-appear with the ring closed and the cabling matching the OBN topology, request Nomad Digital re-verification before further escalation. A regression at this point would indicate a different fault (e.g. transient device flood, m-variant mis-cabling, or a wrong-far-end-port that LLDP cannot detect when neighbour sysName matches expected — none of which are visible in the present measurements).

# 7. Appendix — Tooling and raw evidence

All measurements were performed using read-only SSH sessions from each train's Nomad CCU into the consist switches. The principal commands used were:

- sudo dhcp-lease-list and ip neigh show dev vlan100 — device inventory

- show system, show interface summary, show interface e0-X details — per-switch baseline + counters

- show lldp neighbours — topology verification

- show spanning-tree — RSTP convergence

The expected nv6 topology was extracted programmatically from the OBN templates under nomad-obn-template-nv6/src/etc/obn/template/nv6-*-X.cfg, port descriptions field. The full per-switch raw output (LLDP neighbours, STP state, byte counters, error counters) is retained in the Nomad fleet-status journal (fleet-status.md, train rows 4736-108 and 4736-117, dated 2026-05-29 06:13Z and 06:25Z) and available on request.

Document contact: Abbas Rizvi, abbas.rizvi@nomadrail.com.