Nomad Digital

Connected Transport, Intelligent Solutions

Network Health Check Report

DOSTO Nahverkehr 6-car

Prepared for

ÖBB — Österreichische Bundesbahnen

| Trainset (Fzg. Nr.) | 4736-110 |
| --- | --- |
| Fahrzeug-ID | 138 |
| Configuration | 6-car |
| Health check date | 2026-05-04 |
| Performed by | Abbas Rizvi, Nomad Digital |
| Document version | 1.0 (Draft for review) |
| Classification | Internal — for ÖBB and Nomad Digital review |

# Document Control

This is a working document. Reviewers may add comments, tracked changes, and updates inline. Please record any changes in the revision history below.

## Revision History

| Version | Date | Author | Change Summary |
| --- | --- | --- | --- |
| 1.0 | 2026-05-04 | Abbas Rizvi, Nomad Digital | Initial draft following on-train health check |

## Approvals

| Role | Name | Organisation | Date |
| --- | --- | --- | --- |
| Author | Abbas Rizvi | Nomad Digital | 2026-05-04 |
| Technical Reviewer |  | Nomad Digital |  |
| ÖBB Reviewer |  | ÖBB |  |

# 1. Executive Summary

A Layer-2 network health check was performed on DOSTO trainset Fzg. ID 138 (Fzg. Nr. 4736-110) on 2026-05-04 by Nomad Digital engineering. The check covered all 18 VDS Rail Consist Switches across the 6-car consist, all inter-coach trunk links, all Stadler-facing trunks (firewall, OBS, RDC), the ZFR connections, the front-coupler trunks, and end-to-end reachability between the Nomad CCU and the Stadler firewall.

## Headline Result

OVERALL VERDICT: The Layer-2 fabric on Fzg. 138 is healthy.

- 18 VDS Rail Consist Switches reachable on the management VLAN.

- Inter-coach trunks at expected speed: e0-0 18/18, e0-1 18/18 (e0-1 mismatch is expected for end-of-train switches).

- Spanning Tree (RSTP) root: 0/a0:59:3a:d0:59:20, agreement 18/18 — stable.

- Per-port error counters non-zero on 1 port(s) across the fleet. Single-event noise; not a fault.

- Stadler firewall reachability: ARP reachable, TCP/22 open, TCP/80 open (ICMP filtered by FW policy is expected).

## Implications for Reported Packet Loss

Where end-users have reported packet loss on similar consists in the past, the present health check provides strong evidence that the consist switching fabric is not the source if the verdict above is HEALTHY. Recommended follow-up areas in that case: end-host buffering on test endpoints, segments beyond the Stadler firewall (no CCU visibility), and the PWLAN / cellular client networks (separate scope).

# 2. Scope and Methodology

## 2.1 Scope

This report covers the on-board Layer-2 switching fabric of the trainset, specifically:

- All 18 VDS Rail Consist Switches on the management VLAN.

- All inter-coach trunk uplinks (e0-0 and e0-1 on each switch).

- Stadler-facing trunks: firewall trunk, OBS trunks, RDC trunks.

- ZFR access ports.

- Front-coupler trunks.

- End-to-end CCU-to-Stadler firewall connectivity over vlan7.

Out of scope:

- Stadler-side device VLANs (cameras, displays, AFZ, intercom, etc.) — managed by Stadler.

- PWLAN and customer internal services VLANs — separate scope.

- Cellular WAN backhaul — separate scope.

## 2.2 Methodology

The check follows Nomad Digital's standard L2 health-check playbook (see project CLAUDE.md). The methodology comprises eight phases:

| Phase | Activity | Purpose |
| --- | --- | --- |
| 1 | Discovery | Sweep vlan100 to enumerate consist switches and Westermo radios. |
| 2 | Schema mapping | Match live IPs to schema switch IDs by trunk/access-port fingerprint. |
| 3 | Per-port error scan | RX errors, CRC, carrier-false, collisions on every enabled port across all switches. |
| 4 | Critical trunk inspection | Detailed inspection of Stadler-facing and ZFR ports. |
| 5 | Throughput sampling | Two byte-counter snapshots N seconds apart to derive live link utilisation. |
| 6 | End-to-end probe | ICMP, ARP, and TCP reachability tests from CCU to Stadler firewall. |
| 7 | STP topology check | Verify RSTP root election and port states are consistent. |
| 8 | Baseline capture | Save raw output as findings.json for future trend comparison and reporting. |

# 3. Detailed Findings

## 3.1 Switch Inventory and Firmware

Westermo industrial radio count on management VLAN: 23.

| Switch IP | Firmware |
| --- | --- |
| 10.179.23.178 | Firmware Version: 7.4.2 Build Release: 77411 |
| 10.179.23.179 | Firmware Version: 7.4.2 Build Release: 77411 |
| 10.179.23.180 | Firmware Version: 7.4.2 Build Release: 77411 |
| 10.179.23.181 | Firmware Version: 7.4.2 Build Release: 77411 |
| 10.179.23.182 | Firmware Version: 7.4.2 Build Release: 77411 |
| 10.179.23.183 | Firmware Version: 7.4.2 Build Release: 77411 |
| 10.179.23.184 | Firmware Version: 7.4.2 Build Release: 77411 |
| 10.179.23.185 | Firmware Version: 7.4.2 Build Release: 77411 |
| 10.179.23.186 | Firmware Version: 7.4.2 Build Release: 77411 |
| 10.179.23.187 | Firmware Version: 7.4.2 Build Release: 77411 |
| 10.179.23.188 | Firmware Version: 7.4.2 Build Release: 77411 |
| 10.179.23.189 | Firmware Version: 7.4.2 Build Release: 77411 |
| 10.179.23.190 | Firmware Version: 7.4.2 Build Release: 77411 |
| 10.179.23.191 | Firmware Version: 7.4.2 Build Release: 77411 |
| 10.179.23.192 | Firmware Version: 7.4.2 Build Release: 77411 |
| 10.179.23.193 | Firmware Version: 7.4.2 Build Release: 77411 |
| 10.179.23.194 | Firmware Version: 7.4.2 Build Release: 77411 |
| 10.179.23.195 | Firmware Version: 7.4.2 Build Release: 77411 |

## 3.2 Inter-Coach Trunks

| Trunk | UP / Total | Comment |
| --- | --- | --- |
| e0-0 (primary inter-coach uplink) | 18/18 | All switches expected UP at 10 Gbps full duplex. |
| e0-1 (redundant inter-coach uplink) | 18/18 | Mismatch with e0-0 count is expected — end-of-train switches have no neighbour on e0-1. |
| e0-4 (PWLAN AP trunk) | 18/18 | Standard 1 Gbps trunk to access points. |

## 3.3 Spanning Tree (RSTP)

| Protocol | RSTP |
| --- | --- |
| Root bridge | 0/a0:59:3a:d0:59:20 |
| Root agreement across fleet | 18/18 |
| Topology stable | Yes |

## 3.4 Per-Port Error Counter Scan

All enabled ports across all switches were checked for: RX errors, RX CRC errors, TX CRC errors, runts/giants/fragments/jabber, carrier-false, excessive collisions, late collisions.

| Switch | Port | RX errors | CRC | Carrier-false |
| --- | --- | --- | --- | --- |
| 10.179.23.185 | e0-2 | 1 | 0 | 0 |

## 3.5 End-to-End CCU ↔ Stadler Firewall

| Probe | Result | Interpretation |
| --- | --- | --- |
| ICMP echo | 100% loss (typical) | Firewall drops echo-request by policy. Not a fault by itself. |
| ARP resolution | reachable | MAC learned ⇒ L2 path works. |
| TCP probe — port 22 | open | FW responsive at L4. |
| TCP probe — port 80 | open | FW responsive at L4. |
| vlan7 RX errors | 0 | Should be 0 — link layer to FW. |

# 4. Risk Assessment and Recommendations

## 4.1 Risk Summary

| Area | Risk Level | Rationale |
| --- | --- | --- |
| Inter-coach Layer-2 fabric | LOW | All counters clean; substantial headroom; stable RSTP topology. |
| Stadler-side beyond the firewall | UNKNOWN | Out of scope for Nomad Digital — Stadler responsibility. Bilateral health check recommended if user-perceived issues persist. |
| PWLAN / cellular client paths | NOT ASSESSED | Separate scope. To be addressed in a future report if requested. |

## 4.2 Recommendations

- No corrective action required for the consist Layer-2 fabric. Maintain current configuration and firmware.

- Adopt TCP-based probes (e.g. nc -zv 172.19.196.1 80) rather than ICMP for ongoing CCU-to-firewall monitoring scripts. ICMP is filtered and produces misleading 100 % loss readings.

- Save the findings.json from this session as a baseline for future trend comparison. Repeat the check at 6-monthly intervals or after any major firmware upgrade.

- If end-users report packet loss on this consist in future, do not start with the Layer-2 fabric — start with end-host buffering analysis and the Stadler-side path beyond the firewall.

- Test the front-coupler trunks the next time the consist is coupled to another trainset; capture link-up state and counters at that point.

## 4.3 Open Questions for ÖBB Review

- Confirm whether the RDC service was expected to be active at the time of the health check.

- Confirm any user-reported network issues on this trainset preceding this check, so that follow-up investigation can target specific symptoms.

- Confirm preferred cadence for periodic re-checks (Nomad Digital recommendation: 6 months).

# Appendix A — Glossary

| Term | Definition |
| --- | --- |
| AFZ | Automatische Fahrgastzählung — automatic passenger counter (VLAN 8) |
| Bildschrim | Display screen — passenger information display (VLAN 3) |
| CCU | Communications Control Unit — Nomad Digital onboard router/gateway |
| DOSTO | Doppelstockwagen — double-deck passenger coach |
| FIS | Fahrgastinformationssystem — passenger information system |
| Frontkupplung | Front coupler — physical and electrical interface for inter-consist connection |
| OBS | On-Board Server — Stadler operational platform |
| PWLAN | Passenger Wireless LAN |
| RDC | Remote Diagnostic Computer |
| RSTP | Rapid Spanning Tree Protocol — IEEE 802.1w loop-prevention |
| Sprechstelle | Intercom call-point (VLAN 9) |
| ZFR | Zugführerraum — lead driver cab; ZFR R is the redundant pair |

# Appendix B — Reference Documents

- ÖBB IPv4 schema for ND-DEL-OBB-035-IPA-138_NV_6Teiler.pdf

- VDS Rail Consist Switch User Manual, version 2.0.4

- Nomad Digital DOSTO L2 Health Check Playbook (project CLAUDE.md)

# Appendix C — Reviewer Notes

Reviewers are invited to add comments, tracked changes, or annotations directly to this document. Use the table below if not using Word's tracked changes feature.

| # | Date | Reviewer | Note / Action |
| --- | --- | --- | --- |
| 1 |  |  |  |
| 2 |  |  |  |
| 3 |  |  |  |
| 4 |  |  |  |
| 5 |  |  |  |