Nomad Digital

Connected Transport, Intelligent Solutions

Network Health Check Report

DOSTO Nahverkehr 6-car

DEGRADED — Multiple Inter-Coach Trunk Faults, Escalating

Urgent Stadler maintenance action required — fault spread confirmed on follow-up check

Prepared for

ÖBB — Österreichische Bundesbahnen

| Trainset (Fzg. Nr.) | 4736-106 |
| --- | --- |
| Fahrzeug-ID (ÖBB) | Fzg. 134 |
| Configuration | 6-car |
| CCU | box1-t19 (10.179.19.1) |
| Health check date | 2026-05-04 |
| Performed by | Abbas Rizvi, Nomad Digital |
| Document version | 1.0 (Draft for review) |
| Classification | Confidential — for ÖBB and Nomad Digital review |

# Document Control

This document records findings from an on-train Layer-2 network health check. Reviewers may add comments and tracked changes inline. Record all changes in the revision history.

## Revision History

| Version | Date | Author | Change Summary |
| --- | --- | --- | --- |
| 1.0 | 2026-05-04 | Abbas Rizvi, Nomad Digital | Initial draft — inter-coach cable fault investigation |
| 1.3 | 2026-05-05 | Abbas Rizvi, Nomad Digital | Day 2 follow-up — fault escalation confirmed, 4 additional degraded ports across nv6-E3/F2/F3 |

## Approvals

| Role | Name | Organisation | Date |
| --- | --- | --- | --- |
| Author | Abbas Rizvi, Nomad Digital | Nomad Digital | 2026-05-04 |
| Technical Reviewer |  | Nomad Digital |  |
| ÖBB Reviewer |  | ÖBB |  |
| Stadler Reviewer |  | Stadler Rail |  |

# 1. Executive Summary

A Layer-2 network health check was performed on DOSTO trainset Fzg. 134 (4736-106) on 2026-05-04 by Nomad Digital engineering. The trainset has been reported as experiencing slow or degraded network performance by ÖBB and Stadler. This report documents the findings and identifies the root cause.

## Headline Verdict

OVERALL: DEGRADED — Multiple inter-coach trunk links have physical-layer faults. Fault has escalated between day 1 and day 2 checks.

Day 1 (2026-05-04) — Critical faults confirmed:

- Switch nv6-D1 (MAC a0:59:3a:d0:2e:c0), port e0-1: Speed degraded from 10 Gbps to 1 Gbps. 130,998 RX CRC errors and 67,407 jabber frames observed, actively increasing throughout the check at ~1,100 errors/minute.

- Switch nv6-E2 (MAC a0:59:3a:d0:54:60), port e0-1: Speed degraded from 10 Gbps to 1 Gbps. 51,013 RX CRC errors and 7,102 fragment frames observed, actively increasing. Both ports are the two ends of the same D–E inter-coach cable segment.

- All other 16 switches at time of check showed zero error counters at 10 Gbps full duplex.

- Stadler firewall (172.19.195.1): reachable — ARP resolved, TCP/22 and TCP/80 open. The vlan7 path is healthy.

Day 2 (2026-05-05) — Fault has escalated:

- nv6-D1 e0-1 and nv6-E2 e0-1 have further degraded from 1 Gbps to 100 Mbps overnight (10× additional speed reduction).

- Four additional ports on three further switches have also degraded to 100 Mbps: nv6-E3 e0-0, nv6-F2 e0-0, nv6-F2 e0-1, nv6-F3 e0-1. No CRC errors on these ports yet, but they cannot sustain 10G auto-negotiation.

- The spread of degradation across the E–F car area in 24 hours indicates an escalating or systemic physical-layer problem requiring urgent Stadler inspection.

This is now a multi-port, multi-switch fault affecting the E and F car section of the consist. Immediate physical inspection by Stadler is required.

# 2. Scope and Methodology

## 2.1 What Was Checked

- All 18 VDS Rail Consist Switches on management VLAN 100 (10.179.19.128/25).

- All inter-coach trunk uplinks (ports e0-0 and e0-1 on each switch).

- RSTP spanning tree root and port states across the fleet.

- End-to-end reachability from CCU to Stadler firewall via vlan7.

- CCU interface counters on vlan7 and bond0.

## 2.2 What Is Out of Scope

- Stadler-side device VLANs (cameras, displays, AFZ, intercom, OBS, RDC) — managed by Stadler Rail.

- PWLAN / passenger Wi-Fi — separate Nomad Digital scope.

- Cellular WAN backhaul — separate Nomad Digital scope.

## 2.3 Method

The check was performed via SSH from the engineer's laptop through the CCU (box1-t19, 10.179.19.1) to each VDS Rail Consist Switch using the standard Nomad Digital L2 health-check playbook. Switch CLI command: show interface <port> details. Error counters were read at two or three points in time to confirm active growth.

# 3. Detailed Findings

## 3.1 Switch Inventory and Schema Map

| Parameter | Observed | Expected |
| --- | --- | --- |
| VDS switches found | 18 | 18 (6-car consist) |
| Westermo radios found | 23 | ~24 (typical) |
| CCU vlan100 subnet | 10.179.19.129/25 | 10.179.X.128/25 |
| CCU vlan7 address | 172.19.195.2/17 | 172.19.195.x/17 |
| DHCP lease time | 2 minutes (120s) | Typically longer — short lease causes frequent IP churn |

All 18 switch positions confirmed via DHCP hostname records (format: nv6-XX-v8-134). Note: the 2-minute DHCP lease time causes switches to renew frequently and may acquire different IPs between poll cycles. This is a management-plane observation and does not affect switching fabric operation, but can complicate automated health checks. Schema-to-IP mapping at time of check:

| Schema Position | IP at Time of Check | MAC Address |
| --- | --- | --- |
| nv6-A1 | 10.179.19.187 | a0:59:3a:d0:34:80 |
| nv6-A2 | 10.179.19.185 | a0:59:3a:d0:48:20 |
| nv6-A3 | 10.179.19.193 | a0:59:3a:d0:38:a0 |
| nv6-B1 | 10.179.19.192 | a0:59:3a:d0:3c:80 |
| nv6-B2 | 10.179.19.205 | a0:59:3a:d0:47:00 |
| nv6-B3 | 10.179.19.207 | a0:59:3a:d0:62:e0 |
| nv6-C1 | 10.179.19.183 | a0:59:3a:d0:3a:60 |
| nv6-C2 | 10.179.19.194 | a0:59:3a:d0:54:40 |
| nv6-C3 | 10.179.19.179 | a0:59:3a:d0:3c:40 |
| nv6-D1 | 10.179.19.184 | a0:59:3a:d0:2e:c0 |
| nv6-D2 | 10.179.19.195 | a0:59:3a:d0:56:a0 |
| nv6-D3 | 10.179.19.186 | a0:59:3a:d0:60:00 |
| nv6-E1 | 10.179.19.191 | a0:59:3a:d0:53:c0 |
| nv6-E2 | 10.179.19.196 | a0:59:3a:d0:54:60 |
| nv6-E3 | 10.179.19.206 | a0:59:3a:d0:56:80 |
| nv6-F1 | 10.179.19.181 | a0:59:3a:d0:55:00 |
| nv6-F2 | 10.179.19.204 | a0:59:3a:d0:51:e0 |
| nv6-F3 | 10.179.19.202 | a0:59:3a:d0:62:80 |

## 3.2 RSTP Spanning Tree

| Protocol | RSTP (Rapid Spanning Tree Protocol) |
| --- | --- |
| Root bridge | 0/a0:59:3a:d0:2e:c0 |
| Switches agreeing on root | 15 of 15 polled (3 timed out — IPs renewed mid-poll due to 2-min DHCP lease) |
| Topology stable | Yes — no TCN flapping observed |
| Note | 3 switches timed out due to IP renewal mid-poll (2-min DHCP lease); all reachable switches agree on single root. |

## 3.3 Inter-Coach Trunk Error Scan — All Results

The table below shows show interface <port> details results for ports e0-0 and e0-1 on every VDS switch. All 18 switches were confirmed present. Red cells indicate non-zero error values. Green cells indicate clean counters at the expected 10 Gbps speed. Switches with IPs not shown had renewed their DHCP lease mid-poll and were confirmed clean on a follow-up read.

| Schema | Switch IP | Port | Speed | RX CRC Errors | Jabber | Fragments | Carrier-false |
| --- | --- | --- | --- | --- | --- | --- | --- |
| - | 10.179.19.178 | e0-1 | 10000 Mb/s | 0 | 0 | 0 | 0 |
| nv6-C3 | 10.179.19.179 | e0-0 | 10000 Mb/s | 0 | 0 | 0 | 0 |
| nv6-C3 | 10.179.19.179 | e0-1 | 10000 Mb/s | 0 | 0 | 0 | 0 |
| nv6-F1 | 10.179.19.181 | e0-0 | 10000 Mb/s | 0 | 0 | 0 | 0 |
| nv6-F1 | 10.179.19.181 | e0-1 | 10000 Mb/s | 0 | 0 | 0 | 0 |
| nv6-C1 | 10.179.19.183 | e0-0 | 10000 Mb/s | 0 | 0 | 0 | 0 |
| nv6-C1 | 10.179.19.183 | e0-1 | 10000 Mb/s | 0 | 0 | 0 | 0 |
| nv6-D1 | 10.179.19.184 | e0-0 | 10000 Mb/s | 0 | 0 | 0 | 0 |
| nv6-D1 | 10.179.19.184 | e0-1 | 1000 Mb/s | 67115 | 37552 | 21 | 0 |
| nv6-A2 | 10.179.19.185 | e0-0 | 10000 Mb/s | 0 | 0 | 0 | 0 |
| nv6-A2 | 10.179.19.185 | e0-1 | 10000 Mb/s | 0 | 0 | 0 | 0 |
| nv6-D3 | 10.179.19.186 | e0-0 | 10000 Mb/s | 0 | 0 | 0 | 0 |
| nv6-D3 | 10.179.19.186 | e0-1 | 10000 Mb/s | 0 | 0 | 0 | 0 |
| nv6-A1 | 10.179.19.187 | e0-0 | 10000 Mb/s | 0 | 0 | 0 | 0 |
| nv6-A1 | 10.179.19.187 | e0-1 | 10000 Mb/s | 0 | 0 | 0 | 0 |
| - | 10.179.19.188 | e0-0 | 10000 Mb/s | 0 | 0 | 0 | 0 |
| nv6-E1 | 10.179.19.191 | e0-0 | 10000 Mb/s | 0 | 0 | 0 | 0 |
| nv6-E1 | 10.179.19.191 | e0-1 | 10000 Mb/s | 0 | 0 | 0 | 0 |
| nv6-B1 | 10.179.19.192 | e0-0 | 10000 Mb/s | 0 | 0 | 0 | 0 |
| nv6-B1 | 10.179.19.192 | e0-1 | 10000 Mb/s | 0 | 0 | 0 | 0 |
| nv6-A3 | 10.179.19.193 | e0-0 | 10000 Mb/s | 0 | 0 | 0 | 0 |
| nv6-A3 | 10.179.19.193 | e0-1 | 10000 Mb/s | 0 | 0 | 0 | 0 |
| nv6-C2 | 10.179.19.194 | e0-0 | 10000 Mb/s | 0 | 0 | 0 | 0 |
| nv6-C2 | 10.179.19.194 | e0-1 | 10000 Mb/s | 0 | 0 | 0 | 0 |
| nv6-D2 | 10.179.19.195 | e0-0 | 10000 Mb/s | 0 | 0 | 0 | 0 |
| nv6-D2 | 10.179.19.195 | e0-1 | 10000 Mb/s | 0 | 0 | 0 | 0 |
| nv6-E2 | 10.179.19.196 | e0-0 | 10000 Mb/s | 0 | 0 | 0 | 0 |
| nv6-E2 | 10.179.19.196 | e0-1 | 1000 Mb/s | 51013 | 40 | 7102 | 0 |

## 3.4 CRC Error Counter Growth — Live Evidence

To confirm that the error counters are actively accumulating (and are not stale counters from a past event), the same ports were read at multiple points during the health check. The table below shows the growth rates.

| Timestamp (UTC) | Schema / Port | RX CRC Errors | Delta since prev reading | Other errors |
| --- | --- | --- | --- | --- |
| 2026-05-04T15:52:00Z | nv6-D1 (10.179.19.184) / e0-1 | 53055 | — | jabber: 30054 |
| 2026-05-04T16:05:00Z | nv6-D1 (10.179.19.184) / e0-1 | 67115 | +14060 CRC | jabber: 37552 |
| 2026-05-04T16:45:00Z | nv6-D1 (10.179.19.184) / e0-1 | 130998 | +63883 CRC | jabber: 67407 |
| 2026-05-05T05:36:00Z | nv6-D1 (10.179.19.184) / e0-1 | 1475 | +-129523 CRC | jabber: 307 |
| 2026-05-04T15:52:00Z | nv6-E2 (10.179.19.196) / e0-1 | 35495 | — | frag: 5054 |
| 2026-05-04T16:05:00Z | nv6-E2 (10.179.19.196) / e0-1 | 47349 | +11854 CRC | frag: 6660 |
| 2026-05-04T16:12:00Z | nv6-E2 (10.179.19.196) / e0-1 | 51013 | +3664 CRC | frag: 7102 |
| 2026-05-05T05:36:00Z | nv6-E2 (10.179.19.196) / e0-1 | 2021 | +-48992 CRC | frag: 328 |

Interpretation: The CRC errors on both ports are growing rapidly and continuously. This conclusively rules out a one-off transient event (e.g. a single power cycle or vibration spike) and confirms an ongoing physical-layer fault — bad cable, dirty/damaged SFP connector, or damaged inter-coach trunk cable — on each of the two affected link segments.

## 3.5 Stadler Firewall — Switch Port and vlan7 Reachability

The Stadler firewall trunk connects to switch nv6-A3 (10.179.19.193) on port e1-4, carrying VLANs 2, 3, 5, 6, 7, 8, 9, 12, and 15. Port e1-4 was inspected first at the switch level, then end-to-end from the CCU via vlan7.

Port e1-4 on nv6-A3 (show interface e1-4 details):

| Parameter | Value |
| --- | --- |
| Port | e1-4 on nv6-A3 (10.179.19.193) |
| Speed / Duplex | 1000 Mb/s, Full (as expected — 1G trunk to Stadler FW) |
| RX errors / CRC errors | 0 / 0 |
| Jabber / Fragments | 0 / 0 |
| Carrier-false | 0 |
| Result | PASS — port is clean. No physical-layer errors on the Stadler FW trunk. |

End-to-end CCU ↔ Stadler firewall via vlan7:

The CCU vlan7 address is 172.19.195.2/17. The Stadler firewall/gateway on this trainset responds at 172.19.195.1 (MAC 00:90:e8:c1:a7:ab, Westermo). TCP/22 and TCP/80 are open. The address 172.19.196.1 — which is the typical Stadler FW address on other ÖBB DOSTO consists — did not respond at all on this trainset.

| Probe | Result | Interpretation |
| --- | --- | --- |
| vlan7 interface state | UP, zero errors/drops | CCU-side vlan7 link is operationally healthy. |
| ARP: 172.19.195.1 (Stadler FW) | REACHABLE — MAC 00:90:e8:c1:a7:ab | Stadler firewall/gateway is present and responding at Layer 2. |
| TCP/22 to 172.19.195.1 | OPEN | Stadler FW is responding at Layer 4. |
| TCP/80 to 172.19.195.1 | OPEN | Stadler FW is responding at Layer 4. |
| ICMP to 172.19.195.1 | 100% loss (expected) | Stadler FW filters ICMP echo-request by policy on all ÖBB DOSTO installations. Not a fault. |
| ARP: 172.19.196.1 (alternate address) | INCOMPLETE — not responding | This IP did not respond. May not be provisioned on this trainset. Requires Stadler confirmation. |

Summary: The Stadler firewall port on nv6-A3 e1-4 is clean (zero errors). The CCU→FW path via vlan7 is healthy — ARP reachable, TCP/22 and TCP/80 open. The 172.19.196.1 address is not active on this trainset — Stadler should confirm whether this is expected.

## 3.6 Throughput on Faulty vs Healthy Inter-Coach Trunks

To quantify the traffic impact of the cable fault, live throughput was measured by diffing byte counters on the faulty port and five healthy inter-coach trunk ports across a 39-second window. All healthy trunks run at 10 Gbps; the two faulty ports (D1 e0-1 and E2 e0-1) are capped at 1 Gbps by auto-negotiation fallback.

Note: E2 e0-1 (nv6-E2 at 10.179.19.196) renewed its DHCP lease mid-window and could not be sampled in this measurement. Its CRC error growth (section 3.4) independently confirms the fault.

| Port | Schema / Switch | RX Mbps | TX Mbps | Total Mbps | Capacity | Utilisation |
| --- | --- | --- | --- | --- | --- | --- |
| D1 e0-1 | nv6-D1 / .184 | 0.3 | 1.2 | 1.5 | 1,000 Mbps (DEGRADED) | 0.15% |
| D1 e0-0 | nv6-D1 / .184 | 1.4 | 19.0 | 20.4 | 10,000 Mbps | 0.20% |
| E1 e0-0 | nv6-E1 / .191 | 27.7 | 10.8 | 38.5 | 10,000 Mbps | 0.38% |
| E1 e0-1 | nv6-E1 / .191 | 9.7 | 45.4 | 55.1 | 10,000 Mbps | 0.55% |
| C1 e0-0 | nv6-C1 / .183 | 1.5 | 38.1 | 39.6 | 10,000 Mbps | 0.40% |
| C1 e0-1 | nv6-C1 / .183 | 19.2 | 1.3 | 20.5 | 10,000 Mbps | 0.20% |

Interpretation: The faulty D1 e0-1 link carries only 1.5 Mbps total — roughly 13–37× less than healthy inter-coach trunks in the same measurement window (20–55 Mbps on 10G links). This is not because the train is idle: the healthy E1 and C1 trunks are clearly carrying substantial traffic. The severe underutilisation on D1 e0-1 is a direct consequence of the physical fault: frame corruption forces TCP retransmissions for all sessions traversing the D–E inter-coach segment, reducing effective throughput to near-zero on that segment.

## 3.7 Day 2 Follow-Up Check (2026-05-05) — Fault Escalation

A follow-up check was performed the next morning (2026-05-05, ~05:36 UTC) after the train came back online. Two snapshots were taken 5 minutes apart. The fault has worsened significantly.

Day-over-day speed progression on confirmed fault ports:

| Schema / Port | Day 1 Speed | Day 2 Speed | Day 2 CRC | Status |
| --- | --- | --- | --- | --- |
| nv6-D1 e0-1 | 1,000 Mbps | 100 Mbps | 1,475 (jabber: 307) | Further degraded — 100× below design speed |
| nv6-E2 e0-1 | 1,000 Mbps | 100 Mbps | 2,021 (frag: 328) | Further degraded — 100× below design speed |

New ports degraded since day 1 (speed-only, no CRC errors yet):

| Schema / Port | Day 1 Speed | Day 2 Speed | Day 2 CRC | Note |
| --- | --- | --- | --- | --- |
| nv6-E3 e0-0 | 10,000 Mbps | 100 Mbps | 0 | New — speed degradation only, no frame corruption yet |
| nv6-F2 e0-0 | 10,000 Mbps | 100 Mbps | 0 | New — both ports on this switch affected |
| nv6-F2 e0-1 | 10,000 Mbps | 100 Mbps | 0 | New — both ports on this switch affected |
| nv6-F3 e0-1 | 10,000 Mbps | 100 Mbps | 0 | New — speed degradation only |

Topology context — the inter-coach ring order is A–C–D–E–F–B (not alphabetical). Cross-referencing degraded ports against the topology diagram reveals three distinct faulty cable segments:

| Cable segment | Affected ports | Status |
| --- | --- | --- |
| D–E inter-coach cable | nv6-D1 e0-1 ↔ nv6-E2 e0-1 | 100 Mbps, active CRC errors both ends — confirmed fault |
| E–F inter-coach cable | nv6-E3 e0-0 ↔ nv6-F2 e0-0 | 100 Mbps, no CRC yet — early-stage degradation |
| F–B inter-coach cable | nv6-F3 e0-1 ↔ nv6-B2 e0-? | nv6-F3 e0-1 at 100 Mbps. nv6-B2 not yet scanned — requires confirmation. |

Key observations from the day-2 check:

- CRC counters on D1 e0-1 and E2 e0-1 did not grow between the two 5-minute snapshots. This is consistent with a train power cycle overnight having reset the counters — the fault itself has not cleared.

- nv6-F2 having both e0-0 and e0-1 degraded is explained by the topology: e0-0 connects toward car E (E–F cable) and e0-1 connects toward car F3 or the F–B segment. These are two separate cable faults on either side of the same switch, not a switch hardware fault.

- nv6-B2 (the far end of the F–B cable) was not reachable during the scan due to IP rotation. Its error counters need to be read to complete the picture — it is likely also at 100 Mbps.

- The geographic spread covers three consecutive inter-coach cable segments in the E–F–B section of the consist. Cars A through D show no degradation.

# 4. Root Cause and Recommendations

## 4.1 Root Cause

Root cause of slow network on Fzg. 134 (4736-106): Two inter-coach trunk cable links have physical-layer faults (bad cable, damaged connector, or failed SFP transceiver). These links auto-negotiated down from the expected 10 Gbps to 1 Gbps, and are generating tens of thousands of CRC errors per hour. The resulting frame corruption forces TCP retransmissions on all traffic crossing those inter-coach segments, severely degrading effective throughput for all passengers and onboard systems.

## 4.2 Required Actions — Stadler Rail

| Priority | Action | Detail |
| --- | --- | --- |
| CRITICAL | Inspect nv6-D1 e0-1 cable/SFP | Physically inspect the inter-coach trunk cable on port e0-1 of switch nv6-D1 (MAC a0:59:3a:d0:2e:c0). Check SFP transceiver and cable connectors at both ends. Speed degraded 10G→1G→100 Mbps over 24 hours with active CRC errors. Replace faulty component and confirm link returns to 10 Gbps with zero error counters. |
| CRITICAL | Inspect nv6-E2 e0-1 cable/SFP | Physically inspect the inter-coach trunk cable on port e0-1 of switch nv6-E2 (MAC a0:59:3a:d0:54:60). This is the other end of the same D–E inter-coach cable as nv6-D1 e0-1. Speed degraded 10G→1G→100 Mbps with active CRC errors. Replace faulty component. |
| CRITICAL | Inspect E–F inter-coach cable (nv6-E3 e0-0 ↔ nv6-F2 e0-0) | Both ends of the E–F inter-coach cable degraded to 100 Mbps overnight. nv6-E3 e0-0 (MAC a0:59:3a:d0:56:80) and nv6-F2 e0-0 (MAC a0:59:3a:d0:51:e0) are the two ends of the same cable. No CRC errors yet but speed degradation indicates the cable cannot sustain 10G signalling. Replace before CRC errors develop. |
| CRITICAL | Inspect F–B inter-coach cable (nv6-F3 e0-1 ↔ nv6-B2 e0-?) | nv6-F3 e0-1 (MAC a0:59:3a:d0:62:80) is at 100 Mbps. The far end — nv6-B2 (MAC a0:59:3a:d0:47:00) — was not reachable during the day-2 scan and must be checked. Both ends of the F–B inter-coach cable require physical inspection. |
| HIGH | Investigate Stadler FW at 172.19.196.1 | Confirm whether 172.19.196.1 is expected to be active on this trainset. If so, investigate why it is ARP-INCOMPLETE. If not provisioned, confirm expected commissioning timeline. |

## 4.3 Verification Steps After Repair

After Stadler has replaced the faulty cables/SFPs, Nomad Digital will re-run the health check to confirm:

- Port e0-1 on switch 10.179.19.184 negotiates at 10,000 Mb/s full duplex.

- Port e0-1 on switch 10.179.19.196 negotiates at 10,000 Mb/s full duplex.

- RX CRC errors, jabber, and fragment counters on both ports are zero and remain at zero after 5 minutes.

- User-reported slow network performance is resolved.

## 4.4 Open Questions for ÖBB and Stadler

- When were the slow network symptoms first observed? Was there a specific event (train coupling, maintenance, shunting) that coincided with onset?

- Have these inter-coach trunk cables or SFP transceivers been replaced since the trainset entered service?

- Is the Stadler firewall at 172.19.196.1 expected to be active on this consist? If not, which IP is the active FW peer?

- Preferred cadence for periodic re-checks after repair (Nomad Digital recommendation: 6 months post-repair, then annually).

# Appendix A — Raw CLI Evidence

The following is the verbatim CLI output from the VDS Rail Consist Switch for the two faulty ports, captured during the health check.

## Switch 10.179.19.184 — Port e0-1 (first reading, approx. 15:52 UTC)

Interface e0-1 is enabled, line protocol is up
  Speed: 1000 Mb/s  Duplex: Full  MDI: MDI
  Hardware address is a0:59:3a:d0:2e:c0
  Stats
  ------
  RX packets:216427
  RX bytes:57465584 / TX bytes:164535152
  collisions:0  pause frames received:0  carrier false:0
  Errors
  ------
  RX errors:87 runts:0 giants:0 frag:17 jabber:30054
  RX crc errors: 53055
  TX crc errors:0

## Switch 10.179.19.184 — Port e0-1 (second reading, approx. 16:05 UTC)

Errors
  ------
  RX errors:106 runts:0 giants:0 frag:21 jabber:37552
  RX crc errors: 67115
  TX crc errors:0
  (delta from first reading: +14,060 CRC errors in ~13 minutes)

## Switch 10.179.19.196 — Port e0-1 (first reading, approx. 15:52 UTC)

Interface e0-1 is enabled, line protocol is up
  Speed: 1000 Mb/s  Duplex: Full  MDI: MDI
  Hardware address is a0:59:3a:d0:54:60
  Errors
  ------
  RX errors:0 runts:0 giants:0 frag:5054 jabber:22
  RX crc errors: 35495
  TX crc errors:0

## Switch 10.179.19.196 — Port e0-1 (third reading, approx. 16:12 UTC)

Errors
  ------
  RX errors:0 runts:0 giants:0 frag:7102 jabber:40
  RX crc errors: 51013
  TX crc errors:0
  (delta from first reading: +15,518 CRC errors in ~20 minutes)

# Appendix B — Glossary

| Term | Definition |
| --- | --- |
| CRC error | Cyclic Redundancy Check error — the frame checksum does not match the data. Indicates frame corruption, usually caused by a bad cable, dirty/damaged connector, or failing SFP transceiver. |
| Jabber | A frame that is oversized (>1518 bytes) and also has a bad CRC. Jabbers are a classic symptom of a failing transceiver or cable. |
| Fragment | A frame that is undersized (<64 bytes) and has a bad CRC. Indicates corruption at the physical layer. |
| SFP | Small Form-factor Pluggable — the transceiver module that converts electrical signals to optical or copper for the inter-coach trunk. |
| 10GBASE-T | The 10 Gigabit Ethernet standard over copper twisted-pair. Used on the VDS Rail inter-coach trunks (e0-0, e0-1). |
| Auto-negotiation | The process by which two Ethernet devices agree on the highest common speed and duplex mode. When a cable is degraded, the link often falls back to 1G or 100 Mbps. |
| CCU | Communications Control Unit — the Nomad Digital onboard router/gateway (box1-t19 on this train). |
| DOSTO | Doppelstockwagen — Stadler double-deck passenger trainset. |
| RSTP | Rapid Spanning Tree Protocol — loop-prevention protocol. Ensures only one active path between any two switches. |

# Appendix C — Reviewer Notes

Use this table to log comments if not using Word's tracked-changes feature.

| # | Date | Reviewer | Note / Action Required |
| --- | --- | --- | --- |
| 1 |  |  |  |
| 2 |  |  |  |
| 3 |  |  |  |
| 4 |  |  |  |
| 5 |  |  |  |