Nomad Digital

Connected Transport, Intelligent Solutions

Network Health Check Report

DOSTO Nahverkehr 6-car

Prepared for

OeBB - Oesterreichische Bundesbahnen

| Trainset (Fzg. Nr.) | 4736-108 |
| --- | --- |
| Fahrzeug-ID (OeBB) | Fzg. 136 |
| Configuration | 6-car |
| CCU | box1-t8 (10.179.8.1) |
| Check date | 2026-05-04 |
| Author | Abbas Rizvi, Nomad Digital |
| Version | 1.0 |
| Status | Draft for review |

# Document Control

This document records findings from an on-train Layer-2 network health check performed on DOSTO trainset Fzg. 136 (4736-108) on 2026-05-04. Reviewers may add comments using Word Track Changes or the Reviewer Notes table in Appendix D.

## Revision History

| Version | Date | Author | Change Summary |
| --- | --- | --- | --- |
| 1.0 | 2026-05-04 | Abbas Rizvi, Nomad Digital | Initial draft - routine L2 health check on 4736-108 (Fzg. 136), including OBN config/firmware update and cabling fault assessment. |

## Approvals

| Role | Name | Organisation | Date |
| --- | --- | --- | --- |
| Author | Abbas Rizvi, Nomad Digital | Nomad Digital |  |
| Technical Reviewer |  | Nomad Digital |  |
| OeBB Reviewer |  | OeBB |  |

# 1. Executive Summary

A Layer-2 network health check was performed on DOSTO trainset Fzg. 136 (4736-108) on 2026-05-04 by Abbas Rizvi (Nomad Digital). The check included switch inventory, RSTP topology verification, LLDP cabling audit, per-port error scan, OBN configuration and firmware update, and Westermo access point firmware update. Two cabling faults were identified via LLDP; one resolved by Stadler Rail, one outstanding.

## Headline Verdict

OVERALL VERDICT: CABLING FAULT (one outstanding) - Layer-2 fabric operationally healthy via RSTP redundancy; full clean bill requires Stadler to reconnect the D1<->E2 inter-coach trunk cable.

18 VDS Rail Consist Switches found and reachable on management VLAN.

Inter-coach trunks: e0-0 18/18 UP at 10G full-duplex. e0-1 17/18 UP - D1 e0-1 link-down (missing cable to E2, Stadler action required).

RSTP root bridge: D1 (0/a0:59:3a:d0:5e:c0), 18/18 agreement - single stable root. RSTP routing around the D1<->E2 break; all ports FWD, no traffic loss.

Per-port error scan: 1 port (C2 e0-1) with single-count noise (1x rx_err, 1x jabber, 1x runt, 1x giant - CRC=0, carrier-false=0). Not actionable.

OBN: all 18 VDS switches on nv6-XX-v8-136 and firmware 7.4.2. F3 train_id corrected 144->136.

Westermo APs: firmware 6.10.0-0 -> 6.11.2-0 update in progress (9/24 confirmed at time of report). APxm-v1 config for coaches 4-6 pending.

Cabling fault 1 (C3 e0-0/e0-1 swap): RESOLVED by Stadler - LLDP confirmed correct.

Cabling fault 2 (D1<->E2 missing link): OUTSTANDING - Stadler action required.

# 2. Scope and Methodology

## 2.1 What Was Checked

All 18 VDS Rail Consist Switches on management VLAN 100 (10.179.8.128/25).

All inter-coach trunk uplinks (e0-0 and e0-1 on each switch).

LLDP topology check - e0-0 and e0-1 neighbour mapping vs. OBN expected topology.

RSTP spanning tree root and port states across the fleet.

OBN configuration and firmware audit (nv6-XX-v8-136 target, firmware 7.4.2).

24 Westermo AP firmware update (6.10.0-0 -> 6.11.2-0) and APxm-v1 config push for coaches 4-6.

## 2.2 What Is Out of Scope

Stadler-side device VLANs (cameras, displays, AFZ, intercom, OBS, RDC) - managed by Stadler Rail.

PWLAN / passenger Wi-Fi - separate Nomad Digital scope.

Cellular WAN backhaul - separate Nomad Digital scope.

Full L2 throughput sampling and Stadler FW reachability - deferred until D1<->E2 cabling resolved.

## 2.3 Method

The check was performed via SSH from the engineer laptop through the CCU (box1-t8, 10.179.8.1) to each consist switch. Switch CLI commands (show interface details, show lldp neighbours, show spanning-tree) were run per switch. OBN management used the OBN CLI on the CCU. All 7 known OBN software bugs were pre-patched via fix_obn.py before the update run.

# 3. Detailed Findings

## 3.1 Switch Inventory

| Parameter | Observed | Expected |
| --- | --- | --- |
| VDS switches found | 18 | 18 (6-car consist) |
| Westermo APs found | 24 | 24 (typical 6-car) |
| CCU vlan100 subnet | 10.179.8.128/25 | 10.179.X.128/25 |
| OBN train_id | 136 (= 128 + 8) | Correct for this consist |
| OBN template family | nv6-v7 | nv6-* (6-car) |

## 3.2 Schema-to-IP Mapping

Switches identified by LLDP hostname (nv6-XX-v8-136) after OBN config push. Prior to push, identification was by trunk/access-port fingerprint.

| Schema Role | IP | Notes |
| --- | --- | --- |
| A1 | 10.179.8.185 |  |
| A2 | 10.179.8.194 |  |
| A3 | 10.179.8.182 | Stadler FW switch - e1-4 multi-VLAN trunk |
| C1 | 10.179.8.190 |  |
| C2 | 10.179.8.192 |  |
| C3 | 10.179.8.180 | Cable swap e0-0/e0-1 - now fixed by Stadler |
| D1 | 10.179.8.183 | RSTP root; e0-1 link-down (cable missing to E2) |
| D2 | 10.179.8.195 |  |
| D3 | 10.179.8.186 |  |
| E1 | 10.179.8.179 |  |
| E2 | 10.179.8.191 | e0-1 link-down (cable missing to D1) |
| E3 | 10.179.8.188 |  |
| F1 | 10.179.8.184 |  |
| F2 | 10.179.8.187 |  |
| F3 | 10.179.8.193 | train_id corrected 144->136 |
| B1 | 10.179.8.181 | ZFR primary - e1-11 VLAN 2 |
| B2 | 10.179.8.178 |  |
| B3 | 10.179.8.189 | ZFR standby - e1-11 VLAN 2 |

## 3.3 RSTP Spanning Tree

| Parameter | Value |
| --- | --- |
| Protocol | RSTP (Rapid Spanning Tree Protocol) |
| Root bridge | 0/a0:59:3a:d0:5e:c0 (D1, 10.179.8.183) |
| Switches agreeing on root | 18/18 |
| Topology stable | Yes - single root, no TCN flapping observed |
| D1<->E2 trunk impact | E2 routes to root via E3->F2/F3->B-end (path cost 432100 vs normal 2000). All ports FWD. No traffic loss. |

## 3.4 LLDP Cabling Audit

LLDP topology check run against OBN expected nv6-v7 topology. 36 trunk ports checked (e0-0 and e0-1 on all 18 switches).

| Switch | Port | Live Neighbour | Expected | Status |
| --- | --- | --- | --- | --- |
| C3 (10.179.8.180) | e0-0 | A2 | A2 | RESOLVED |
| C3 (10.179.8.180) | e0-1 | C2 | C2 | RESOLVED |
| D1 (10.179.8.183) | e0-1 | NO NEIGHBOUR | E2 | OUTSTANDING |
| E2 (10.179.8.191) | e0-1 | NO NEIGHBOUR | D1 | OUTSTANDING |

All other 32 trunk ports: LLDP neighbours match OBN expected topology.

## 3.5 Inter-Coach Trunk Error Scan

All 36 inter-coach trunk ports checked. D1 e0-1 and E2 e0-1 are link-down (cable missing - no flapping, no errors). One noise event on C2 e0-1 (CRC=0, carrier-false=0 - not actionable).

| Schema | Switch IP | Port | Speed | RX Errors | RX CRC | Jabber/Carrier-false |
| --- | --- | --- | --- | --- | --- | --- |
| B2 | 10.179.8.178 | e0-0 | 10000 Mb/s, Full | 0 | 0 | 0 / 0 |
| B2 | 10.179.8.178 | e0-1 | 10000 Mb/s, Full | 0 | 0 | 0 / 0 |
| E1 | 10.179.8.179 | e0-0 | 10000 Mb/s, Full | 0 | 0 | 0 / 0 |
| E1 | 10.179.8.179 | e0-1 | 10000 Mb/s, Full | 0 | 0 | 0 / 0 |
| C3 | 10.179.8.180 | e0-0 | 10000 Mb/s, Full | 0 | 0 | 0 / 0 |
| C3 | 10.179.8.180 | e0-1 | 10000 Mb/s, Full | 0 | 0 | 0 / 0 |
| B1 | 10.179.8.181 | e0-0 | 10000 Mb/s, Full | 0 | 0 | 0 / 0 |
| B1 | 10.179.8.181 | e0-1 | 10000 Mb/s, Full | 0 | 0 | 0 / 0 |
| A3 | 10.179.8.182 | e0-0 | 10000 Mb/s, Full | 0 | 0 | 0 / 0 |
| A3 | 10.179.8.182 | e0-1 | 10000 Mb/s, Full | 0 | 0 | 0 / 0 |
| D1 | 10.179.8.183 | e0-0 | 10000 Mb/s, Full | 0 | 0 | 0 / 0 |
| D1 | 10.179.8.183 | e0-1 | link DOWN (cable missing) | 0 | 0 | 0 |
| F1 | 10.179.8.184 | e0-0 | 10000 Mb/s, Full | 0 | 0 | 0 / 0 |
| F1 | 10.179.8.184 | e0-1 | 10000 Mb/s, Full | 0 | 0 | 0 / 0 |
| A1 | 10.179.8.185 | e0-0 | 10000 Mb/s, Full | 0 | 0 | 0 / 0 |
| A1 | 10.179.8.185 | e0-1 | 10000 Mb/s, Full | 0 | 0 | 0 / 0 |
| D3 | 10.179.8.186 | e0-0 | 10000 Mb/s, Full | 0 | 0 | 0 / 0 |
| D3 | 10.179.8.186 | e0-1 | 10000 Mb/s, Full | 0 | 0 | 0 / 0 |
| F2 | 10.179.8.187 | e0-0 | 10000 Mb/s, Full | 0 | 0 | 0 / 0 |
| F2 | 10.179.8.187 | e0-1 | 10000 Mb/s, Full | 0 | 0 | 0 / 0 |
| E3 | 10.179.8.188 | e0-0 | 10000 Mb/s, Full | 0 | 0 | 0 / 0 |
| E3 | 10.179.8.188 | e0-1 | 10000 Mb/s, Full | 0 | 0 | 0 / 0 |
| B3 | 10.179.8.189 | e0-0 | 10000 Mb/s, Full | 0 | 0 | 0 / 0 |
| B3 | 10.179.8.189 | e0-1 | 10000 Mb/s, Full | 0 | 0 | 0 / 0 |
| C1 | 10.179.8.190 | e0-0 | 10000 Mb/s, Full | 0 | 0 | 0 / 0 |
| C1 | 10.179.8.190 | e0-1 | 10000 Mb/s, Full | 0 | 0 | 0 / 0 |
| E2 | 10.179.8.191 | e0-0 | 10000 Mb/s, Full | 0 | 0 | 0 / 0 |
| E2 | 10.179.8.191 | e0-1 | link DOWN (cable missing) | 0 | 0 | 0 |
| C2 | 10.179.8.192 | e0-0 | 10000 Mb/s, Full | 0 | 0 | 0 / 0 |
| C2 | 10.179.8.192 | e0-1 | 10000 Mb/s, Full | 1 (noise) | 0 | 1 / 0 |
| F3 | 10.179.8.193 | e0-0 | 10000 Mb/s, Full | 0 | 0 | 0 / 0 |
| F3 | 10.179.8.193 | e0-1 | 10000 Mb/s, Full | 0 | 0 | 0 / 0 |
| A2 | 10.179.8.194 | e0-0 | 10000 Mb/s, Full | 0 | 0 | 0 / 0 |
| A2 | 10.179.8.194 | e0-1 | 10000 Mb/s, Full | 0 | 0 | 0 / 0 |
| D2 | 10.179.8.195 | e0-0 | 10000 Mb/s, Full | 0 | 0 | 0 / 0 |
| D2 | 10.179.8.195 | e0-1 | 10000 Mb/s, Full | 0 | 0 | 0 / 0 |

## 3.6 OBN Configuration and Firmware

All 18 VDS switches updated to nv6-XX-v8-136 (config) and 7.4.2 (firmware). F3 template error corrected (train_id 144->136). All 7 OBN software bugs pre-patched via fix_obn.py.

| Switch | IP | Config | Firmware |
| --- | --- | --- | --- |
| A1 | 10.179.8.185 | nv6-A1-v8-136 | 7.4.2 |
| A2 | 10.179.8.194 | nv6-A2-v8-136 | 7.4.2 |
| A3 | 10.179.8.182 | nv6-A3-v8-136 | 7.4.2 |
| C1 | 10.179.8.190 | nv6-C1-v8-136 | 7.4.2 |
| C2 | 10.179.8.192 | nv6-C2-v8-136 | 7.4.2 |
| C3 | 10.179.8.180 | nv6-C3-v8-136 | 7.4.2 |
| D1 | 10.179.8.183 | nv6-D1-v8-136 | 7.4.2 |
| D2 | 10.179.8.195 | nv6-D2-v8-136 | 7.4.2 |
| D3 | 10.179.8.186 | nv6-D3-v8-136 | 7.4.2 |
| E1 | 10.179.8.179 | nv6-E1-v8-136 | 7.4.2 |
| E2 | 10.179.8.191 | nv6-E2-v8-136 | 7.4.2 |
| E3 | 10.179.8.188 | nv6-E3-v8-136 | 7.4.2 |
| F1 | 10.179.8.184 | nv6-F1-v8-136 | 7.4.2 |
| F2 | 10.179.8.187 | nv6-F2-v8-136 | 7.4.2 |
| F3 | 10.179.8.193 | nv6-F3-v8-136 (corrected from 144) | 7.4.2 |
| B1 | 10.179.8.181 | nv6-B1-v8-136 | 7.4.2 |
| B2 | 10.179.8.178 | nv6-B2-v8-136 | 7.4.2 |
| B3 | 10.179.8.189 | nv6-B3-v8-136 | 7.4.2 |

## 3.7 Westermo AP Firmware Update

Firmware update 6.10.0-0 -> 6.11.2-0 initiated for all 24 APs. At time of report: 9/24 confirmed on 6.11.2-0. APxm-v1 config push to coaches 4-6 (12 APs) pending firmware completion.

| Coach | AP | IP | Firmware Status |
| --- | --- | --- | --- |
| 1 | 1 | 10.179.8.240 | 6.11.2-0 |
| 1 | 3 | 10.179.8.231 | 6.11.2-0 |
| 1 | 4 | 10.179.8.223 | Update sent - completing |
| 2 | 1 | 10.179.8.236 | 6.11.2-0 |
| 2 | 2 | 10.179.8.233 | 6.11.2-0 |
| 2 | 3 | 10.179.8.232 | 6.11.2-0 |
| 2 | 4 | 10.179.8.225 | 6.11.2-0 |
| 3 | 1 | 10.179.8.218 | 6.11.2-0 |
| 3 | 2 | 10.179.8.238 | Update sent - completing |
| 3 | 3 | 10.179.8.226 | Update sent - completing |
| 3 | 4 | 10.179.8.220 | Update sent - completing |
| 4 | 1 | 10.179.8.239 | Update sent - completing |
| 4 | 2 | 10.179.8.235 | Update sent - completing |
| 4 | 3 | 10.179.8.227 | Update sent - completing |
| 4 | 4 | 10.179.8.224 | Update sent - completing |
| 5 | 1 | 10.179.8.234 | Update sent - completing |
| 5 | 2 | 10.179.8.229 | Update sent - completing |
| 5 | 3 | 10.179.8.222 | Update sent - completing |
| 5 | 4 | 10.179.8.219 | Update sent - APxm-v1 config pending |
| 6 | 1 | 10.179.8.230 | Update sent - APxm-v1 config pending |
| 6 | 2 | 10.179.8.237 | Update sent - APxm-v1 config pending |
| 6 | 3 | 10.179.8.228 | Update sent - APxm-v1 config pending |
| 6 | 4 | 10.179.8.221 | Update sent - APxm-v1 config pending |

# 4. Risk Assessment and Recommendations

## 4.1 Risk Summary

| Area | Risk Level | Rationale |
| --- | --- | --- |
| Inter-coach Layer-2 fabric | LOW | All error counters clean. RSTP stable, single root. |
| D1<->E2 missing cable | MEDIUM | One inter-coach trunk broken. RSTP provides redundancy but resilience is reduced. Stadler action required. |
| AP firmware rollout | LOW | 15/24 APs completing update at time of report. PWLAN may be degraded until complete. |
| Stadler-side beyond FW | UNKNOWN | Out of scope - Stadler Rail manages those VLANs. |
| PWLAN / cellular | NOT ASSESSED | Separate scope. |

## 4.2 Recommendations

Stadler Rail: reconnect cable between D1 port e0-1 and E2 port e0-1. After fix: run sudo obn discover && sudo obn validate on the CCU to confirm LLDP topology clean.

Nomad Digital: once all 24 APs confirmed on 6.11.2-0, push APxm-v1 config to coaches 4-6 IPs.

Once cabling resolved: perform full L2 health check (STP re-verify, error counters, Stadler FW reachability, throughput sampling).

Use these findings as baseline for future checks. Repeat at 6-monthly intervals or after any maintenance.

If end-users report packet loss, the Layer-2 fabric is confirmed healthy (D1<->E2 exception aside). Investigate PWLAN/cellular or Stadler-side.

## 4.3 Open Questions for OeBB / Stadler

Stadler Rail: when will the D1 e0-1 <-> E2 e0-1 cable be reconnected?

OeBB: confirm preferred cadence for periodic L2 re-checks (recommendation: 6 months).

OeBB: confirm whether RDC service was expected active at time of check.

OeBB: confirm any user-reported network issues on this trainset for targeted follow-up.

# Appendix A - Raw CLI Evidence

Verbatim CLI output for key ports.

## C2 (10.179.8.192) - e0-1 noise event

Interface e0-1 is enabled, line protocol is up
  Speed: 10000 Mb/s  Duplex: Full
  RX packets:90963984  TX packets:63434312
  RX errors:1 runts:1 giants:1 frag:0 jabber:1
  RX crc errors: 0  TX crc errors:0  carrier false:0

Note: 1 RX error against ~91M packets, CRC=0, carrier-false=0. Single corrupted frame at link init - noise, not actionable. C2 e0-1 connects to D3.

## D1 (10.179.8.183) - e0-1 broken link

Interface e0-1 is enabled, line protocol is down
  Speed: Auto  Duplex: Auto
  RX packets:0  TX packets:0
  RX errors:0 runts:0 giants:0 jabber:0
  RX crc errors: 0  carrier false:0

Note: Link simply down - no errors, no flapping. Cable physically disconnected.

## C3 (10.179.8.180) - LLDP after cable swap fix

e0-0  a0:59:3a:d0:54:20  nv6-A2-v8-136  TTCMP bridge/switch
e0-1  a0:59:3a:d0:63:00  nv6-C2-v8-136  TTCMP bridge/switch

Note: e0-0 -> A2 (expected), e0-1 -> C2 (expected). Cabling fault resolved.

# Appendix B - Glossary

| Term | Definition |
| --- | --- |
| AFZ | Automatische Fahrgastzahlung - automatic passenger counting |
| CCU | Communications Control Unit - Nomad Digital onboard server (Debian Linux) |
| DOSTO | Doppelstockwagen - Stadler Rail double-deck passenger train |
| Fzg. | Fahrzeug - German for vehicle/trainset |
| LLDP | Link Layer Discovery Protocol - L2 neighbour discovery |
| OBN | Onboard Network - Nomad Digital network management software on the CCU |
| RDC | Remote Diagnostic Centre - Stadler remote diagnostics |
| RSTP | Rapid Spanning Tree Protocol - L2 loop prevention and redundancy |
| VDS Rail | VDS Rail GmbH - manufacturer of consist switches on DOSTO trainsets |
| ZFR | Zugfunk Radio - onboard radio; B1/B3 e1-11 access port VLAN 2 |

# Appendix C - Reference Documents

OeBB IPv4 schema: ND-DEL-OBB-035-IPA-136_NV_6Teiler.pdf (Fzg. 136, 4736-108)

VDS Rail Consist Switch User Manual, version 2.0.4

Nomad Digital DOSTO L2 Health Check Playbook (project CLAUDE.md)

Nomad Digital fix_obn.py - OBN bug fix script (bugs 1-7)

Stadler_4736-108_Cabling_Fault_Report_v1.0.docx - prior cabling fault report

# Appendix D - Reviewer Notes

Use this table to log comments if not using Word Track Changes.

| # | Date | Reviewer | Note / Action Required |
| --- | --- | --- | --- |
| 1 |  |  |  |
| 2 |  |  |  |
| 3 |  |  |  |
| 4 |  |  |  |
| 5 |  |  |  |