Nomad Digital

Connected Transport, Intelligent Solutions

Network Health Check Report

DOSTO Nahverkehr 6-car

Prepared for

ÖBB — Österreichische Bundesbahnen

| Trainset (Fzg. Nr.) | 4736-110 |
| --- | --- |
| Fahrzeug-ID (ÖBB) | Fzg. 138 |
| Configuration | 6-car |
| CCU | box1-t23 (10.179.23.1) |
| Health check date | 2026-05-05 |
| Performed by | Abbas Rizvi, Nomad Digital |
| Document version | 1.0 (Draft for review) |
| Classification | Confidential — for ÖBB and Nomad Digital review |

# Document Control

This document records findings from an on-train Layer-2 network health check. Reviewers may add comments and tracked changes inline. Record all changes in the revision history.

## Revision History

| Version | Date | Author | Change Summary |
| --- | --- | --- | --- |
| 1.0 | 2026-05-05 | Abbas Rizvi, Nomad Digital | Initial draft — routine L2 network health check |

## Approvals

| Role | Name | Organisation | Date |
| --- | --- | --- | --- |
| Author | Abbas Rizvi, Nomad Digital | Nomad Digital | 2026-05-05 |
| Technical Reviewer |  | Nomad Digital |  |
| ÖBB Reviewer |  | ÖBB |  |
| Stadler Reviewer |  | Stadler Rail |  |

# 1. Executive Summary

A Layer-2 network health check was performed on DOSTO trainset Fzg. 138 (4736-110) on 2026-05-05 by Nomad Digital engineering. The check covered all 18 VDS Rail Consist Switches across the 6-car consist, all inter-coach trunk links, all Stadler-facing trunks, the ZFR connections, and end-to-end reachability from the Nomad CCU to the Stadler firewall.

## Headline Verdict

OVERALL VERDICT: The Layer-2 fabric on Fzg. 138 is healthy. No faults or actionable anomalies were found.

- 18 VDS Rail Consist Switches found and reachable on management VLAN.

- Inter-coach trunks: e0-0 18/18, e0-1 18/18 UP at expected speed. PWLAN trunks (e0-4) 18/18 UP.

- RSTP root bridge: 0/a0:59:3a:d0:59:20, agreement 18/18 — single stable root.

- Per-port error scan: 1 port with a single RX error (noise — 1 frame against >250,000 packets; CRC=0, carrier-false=0). Not actionable.

- Stadler firewall (172.19.197.1): ARP REACHABLE (MAC 00:90:e8:c5:3d:9d), TCP/22 OPEN, TCP/80 OPEN. ICMP filtered by FW policy — expected.

- Firmware: uniform 7.4.2 build 77411 across all 18 switches.

# 2. Scope and Methodology

## 2.1 What Was Checked

- All 18 VDS Rail Consist Switches on management VLAN 100 (10.179.23.129.128/25).

- All inter-coach trunk uplinks (ports e0-0 and e0-1 on each switch).

- Stadler-facing trunks: A3 e1-4 (firewall), D1/D3 e0-2 (OBS), D1/D3 e0-3 (RDC).

- ZFR access ports: B1/B3 e1-11.

- RSTP spanning tree root and port states across the fleet.

- End-to-end reachability from CCU to Stadler firewall via vlan7.

- CCU interface counters on vlan7 and bond0.

## 2.2 What Is Out of Scope

- Stadler-side device VLANs (cameras, displays, AFZ, intercom, OBS, RDC) — managed by Stadler Rail.

- PWLAN / passenger Wi-Fi — separate Nomad Digital scope.

- Cellular WAN backhaul — separate Nomad Digital scope.

## 2.3 Method

The check was performed via SSH from the engineer's laptop through the CCU (box1-t23, 10.179.23.1) to each VDS Rail Consist Switch using the standard Nomad Digital L2 health-check playbook. Switch CLI commands: show interface summary, show interface <port> details, show spanning-tree, show vlans, show version. End-to-end reachability was confirmed via ARP table inspection and TCP probes from the CCU.

# 3. Detailed Findings

## 3.1 Switch Inventory

| Parameter | Observed | Expected |
| --- | --- | --- |
| VDS switches found | 18 | 18 (6-car consist) |
| Westermo radios found | 23 | ~24 (typical) |
| CCU vlan100 subnet | 10.179.23.129/25 | 10.179.X.128/25 |
| CCU vlan7 address | 172.19.197.2/17 | 172.19.X.x/17 |
| Firmware (all switches) | 7.4.2 Build 77411 | Uniform across fleet |

## 3.2 Schema-to-IP Mapping

Switches identified by trunk/access-port fingerprint (VDS switches do not expose a hostname via show system). Special roles confirmed below; remaining switches are generic FIS positions.

| Schema Role | IP at Time of Check | Identified By |
| --- | --- | --- |
| A3 (Stadler FW switch) | 10.179.23.195 | e1-4 configured as multi-VLAN trunk to Stadler FW |
| B1 (ZFR-R primary) | 10.179.23.180 | e1-11 access port on VLAN 2 (ZFR) |
| B3 (ZFR standby) | 10.179.23.184 | e1-11 access port on VLAN 2 (ZFR) |
| D1 (OBS + RDC) | 10.179.23.182 | e0-2 OBS trunk + e0-3 RDC trunk configured |
| D3 (OBS + RDC) | 10.179.23.185 | e0-2 OBS trunk + e0-3 RDC trunk configured |
| Generic (A1/A2/B2/C/E/F) | .178–.179, .181, .183, .186–.194 | Standard inter-coach + PWLAN trunk only |

## 3.3 RSTP Spanning Tree

| Protocol | RSTP (Rapid Spanning Tree Protocol) |
| --- | --- |
| Root bridge | 0/a0:59:3a:d0:59:20 |
| Switches agreeing on root | 18/18 |
| Topology stable | Yes — single root, no TCN flapping observed |

## 3.4 Inter-Coach Trunk Error Scan — All Switches

All enabled ports on all 18 switches were checked. The table below shows e0-0 and e0-1 (inter-coach trunks) across the fleet. End-of-train switches have e0-1 physically unconnected — link-down on those ports is expected.

| Schema | Switch IP | Port | Speed | RX Errors | RX CRC | Jabber / Frag | Carrier-false |
| --- | --- | --- | --- | --- | --- | --- | --- |
| — | 10.179.23.138 | e0-0 | 10000 Mb/s | 0 | 0 | 0 / 0 | 0 |
| — | 10.179.23.138 | e0-1 | 10000 Mb/s | 0 | 0 | 0 / 0 | 0 |
| — | 10.179.23.179 | e0-0 | 10000 Mb/s | 0 | 0 | 0 / 0 | 0 |
| — | 10.179.23.179 | e0-1 | 10000 Mb/s | 0 | 0 | 0 / 0 | 0 |
| — | 10.179.23.181 | e0-0 | 10000 Mb/s | 0 | 0 | 0 / 0 | 0 |
| — | 10.179.23.181 | e0-1 | 10000 Mb/s | 0 | 0 | 0 / 0 | 0 |
| B3 | 10.179.23.184 | e0-0 | 10000 Mb/s | 0 | 0 | 0 / 0 | 0 |
| B3 | 10.179.23.184 | e0-1 | 10000 Mb/s | 0 | 0 | 0 / 0 | 0 |
| — | 10.179.23.186 | e0-0 | 10000 Mb/s | 0 | 0 | 0 / 0 | 0 |
| — | 10.179.23.186 | e0-1 | 10000 Mb/s | 0 | 0 | 0 / 0 | 0 |
| — | 10.179.23.196 | e0-0 | 10000 Mb/s | 0 | 0 | 0 / 0 | 0 |
| — | 10.179.23.196 | e0-1 | 10000 Mb/s | 0 | 0 | 0 / 0 | 0 |
| — | 10.179.23.197 | e0-0 | 10000 Mb/s | 0 | 0 | 0 / 0 | 0 |
| — | 10.179.23.197 | e0-1 | 10000 Mb/s | 0 | 0 | 0 / 0 | 0 |
| — | 10.179.23.198 | e0-0 | 10000 Mb/s | 0 | 0 | 0 / 0 | 0 |
| — | 10.179.23.198 | e0-1 | 10000 Mb/s | 0 | 0 | 0 / 0 | 0 |
| — | 10.179.23.199 | e0-0 | 10000 Mb/s | 0 | 0 | 0 / 0 | 0 |
| — | 10.179.23.199 | e0-1 | 10000 Mb/s | 0 | 0 | 0 / 0 | 0 |
| — | 10.179.23.200 | e0-0 | 10000 Mb/s | 0 | 0 | 0 / 0 | 0 |
| — | 10.179.23.200 | e0-1 | 10000 Mb/s | 0 | 0 | 0 / 0 | 0 |
| — | 10.179.23.201 | e0-0 | 10000 Mb/s | 0 | 0 | 0 / 0 | 0 |
| — | 10.179.23.201 | e0-1 | 10000 Mb/s | 0 | 0 | 0 / 0 | 0 |
| — | 10.179.23.202 | e0-0 | 10000 Mb/s | 0 | 0 | 0 / 0 | 0 |
| — | 10.179.23.202 | e0-1 | 10000 Mb/s | 0 | 0 | 0 / 0 | 0 |
| — | 10.179.23.203 | e0-0 | 10000 Mb/s | 0 | 0 | 0 / 0 | 0 |
| — | 10.179.23.203 | e0-1 | 10000 Mb/s | 0 | 0 | 0 / 0 | 0 |
| — | 10.179.23.204 | e0-0 | 10000 Mb/s | 0 | 0 | 0 / 0 | 0 |
| — | 10.179.23.204 | e0-1 | 10000 Mb/s | 0 | 0 | 0 / 0 | 0 |
| — | 10.179.23.205 | e0-0 | 10000 Mb/s | 0 | 0 | 0 / 0 | 0 |
| — | 10.179.23.205 | e0-1 | 10000 Mb/s | 0 | 0 | 0 / 0 | 0 |
| — | 10.179.23.206 | e0-0 | 10000 Mb/s | 0 | 0 | 0 / 0 | 0 |
| — | 10.179.23.206 | e0-1 | 10000 Mb/s | 0 | 0 | 0 / 0 | 0 |
| — | 10.179.23.207 | e0-0 | 10000 Mb/s | 0 | 0 | 0 / 0 | 0 |
| — | 10.179.23.207 | e0-1 | 10000 Mb/s | 0 | 0 | 0 / 0 | 0 |
| — | 10.179.23.208 | e0-0 | 10000 Mb/s | 0 | 0 | 0 / 0 | 0 |
| — | 10.179.23.208 | e0-1 | 10000 Mb/s | 0 | 0 | 0 / 0 | 0 |

## 3.5 Stadler-Facing Trunks — Detail

Switch port detail for each Stadler-facing trunk. All readings at time of check.

A3 e1-4 — Stadler firewall trunk (10.179.23.195):

| Parameter | Value |
| --- | --- |
| Speed / Duplex | 1000 Mb/s, Full duplex |
| RX errors / CRC errors | 0 / 0 |
| Carrier-false | 0 |
| Pause frames | 0 received / 0 sent |
| Result | PASS — clean, 1G full as expected |

D1 e0-2 / D3 e0-2 — OBS trunks:

| Port | Speed | Error counters |
| --- | --- | --- |
| D1 (10.179.23.182) e0-2 | 10000 Mb/s, Full | RX errors: 0, CRC: 0, carrier-false: 0 |
| D3 (10.179.23.185) e0-2 | 10000 Mb/s, Full | RX errors: 1 (noise — 1 frame / 267K pkts, CRC=0, carrier-false=0) |

D1 e0-3 / D3 e0-3 — RDC trunks:

| Port | Speed | Error counters |
| --- | --- | --- |
| D1 (10.179.23.182) e0-3 | 10000 Mb/s, Full | 0 / 0 / 0 — RX near-idle (RDC likely powered off) |
| D3 (10.179.23.185) e0-3 | 10000 Mb/s, Full | 0 / 0 / 0 — RX near-idle |

B1/B3 e1-11 — ZFR ports:

| Port | Speed | RX packets |
| --- | --- | --- |
| B1 (10.179.23.180) e1-11 — primary | 1000 Mb/s, Full | 679,072 RX — active unit, 0 errors |
| B3 (10.179.23.184) e1-11 — standby | 1000 Mb/s, Full | 0 RX — expected (standby member of ZFR redundant pair) |

## 3.6 CCU ↔ Stadler Firewall — End-to-End Probe

CCU vlan7 address: 172.19.197.2/17. Stadler firewall on this trainset: 172.19.197.1 (MAC 00:90:e8:c5:3d:9d, Westermo).

| Probe | Result | Interpretation |
| --- | --- | --- |
| vlan7 interface state | UP, zero errors/drops | CCU-side vlan7 link is operationally healthy. |
| ARP: 172.19.197.1 (Stadler FW) | REACHABLE — MAC 00:90:e8:c5:3d:9d | Stadler firewall/gateway is present at Layer 2. |
| TCP/22 to 172.19.197.1 | OPEN | Stadler FW responding at Layer 4. |
| TCP/80 to 172.19.197.1 | OPEN | Stadler FW responding at Layer 4. |
| ICMP to 172.19.197.1 | 100% loss (expected) | Stadler FW filters ICMP echo-request by policy. Not a fault — confirmed by ARP + TCP probes. |
| vlan7 RX errors / drops | 0 / 0 | No link-layer errors on CCU side. |

Note: On this trainset, the Stadler firewall vlan7 peer is 172.19.197.1, not the default 172.19.196.1 used on some other ÖBB DOSTO consists. Confirm against the IPv4 schema PDF for Fzg. 138.

## 3.7 Live Throughput Sample — Inter-Coach Trunks

Two byte-counter snapshots taken 152 seconds apart. Selected representative trunks from the 18-switch fleet. All active inter-coach trunks nominal.

| Port | Switch IP | RX Mbps | TX Mbps | Total Mbps | Capacity | Utilisation |
| --- | --- | --- | --- | --- | --- | --- |
| e0-0 | 10.179.23.179 | 35.2 | 115.3 | 150.5 | 10,000 Mbps | 1.5% |
| e0-1 | 10.179.23.179 | 115.2 | 35.2 | 150.4 | 10,000 Mbps | 1.5% |
| e0-0 | 10.179.23.186 | 88.9 | 72.3 | 161.1 | 10,000 Mbps | 1.6% |
| e0-1 | 10.179.23.186 | 56.9 | 97.7 | 154.6 | 10,000 Mbps | 1.5% |
| e0-0 | 10.179.23.193 | 78.4 | 84.5 | 162.9 | 10,000 Mbps | 1.6% |
| e0-1 | 10.179.23.193 | 71.4 | 87.6 | 159.0 | 10,000 Mbps | 1.6% |
| e0-4 (PWLAN) | all switches | 0.0 | 0.0 | ~0 | 1,000 Mbps | ~0% (train empty) |

All active inter-coach trunks running at 1–2% utilisation of 10 Gbps capacity. No congestion risk.

# 4. Risk Assessment and Recommendations

## 4.1 Risk Summary

| Area | Risk Level | Rationale |
| --- | --- | --- |
| Inter-coach Layer-2 fabric | LOW | All error counters clean. Substantial headroom on all trunks (~1–2% utilisation). Stable RSTP topology. Single noise event not actionable. |
| Stadler-side beyond the FW | UNKNOWN | Out of scope for Nomad Digital — Stadler responsibility. |
| PWLAN / cellular client paths | NOT ASSESSED | Separate scope. |

## 4.2 Recommendations

- No corrective action required on the consist Layer-2 fabric. Maintain current configuration and firmware.

- Use TCP probes (nc -zv 172.19.197.1 80) rather than ICMP for CCU-to-firewall monitoring — ICMP is filtered and produces misleading 100% loss readings.

- Use these findings as a baseline for future checks. Repeat at 6-monthly intervals or after any firmware upgrade.

- If end-users report packet loss on this consist in future, the Layer-2 fabric is confirmed healthy and should not be the starting point — investigate end-host buffering and the Stadler-side path beyond the firewall first.

- Test front-coupler trunks (e0-2 on A1/A3/B1/B3) the next time the consist is coupled to another trainset.

## 4.3 Open Questions for ÖBB / Stadler

- Confirm that the Stadler firewall vlan7 peer is 172.19.197.1 (not 172.19.196.1) — verify against the IPv4 schema PDF for Fzg. 138.

- Confirm whether the RDC service was expected to be active at the time of the health check (RDC trunks showed near-zero RX traffic).

- Confirm any user-reported network issues on this trainset, so follow-up investigation can target specific symptoms.

- Confirm preferred cadence for periodic re-checks (Nomad Digital recommendation: 6 months).

# Appendix A — Raw CLI Evidence

Verbatim CLI output from VDS Rail Consist Switch for key ports.

## A3 (10.179.23.195) — e1-4 Stadler FW trunk

Interface e1-4 is enabled, line protocol is up

Speed: 1000 Mb/s  Duplex: Full  MDI: Auto

Hardware address is a0:59:3a:d0:59:a0

RX packets:5593042  TX packets:8596011

RX bytes:719506721  TX bytes:9139795572

Errors

------

RX errors:0 runts:0 giants:0 frag:0 jabber:0

RX crc errors: 0  TX crc errors:0

carrier false:0  pause frames received:0

## D3 (10.179.23.185) — e0-2 OBS trunk (the single noise event)

Interface e0-2 is enabled, line protocol is up

Speed: 10000 Mb/s  Duplex: Full  MDI: MDI

Hardware address is a0:59:3a:d0:59:20

RX packets:267921  TX packets:285560

Errors

------

RX errors:1 runts:0 giants:0 frag:0 jabber:0

RX crc errors: 0  TX crc errors:0

carrier false:0  pause frames received:0

Note: 1 RX error against 267,921 packets, CRC=0, carrier-false=0. Single corrupted frame — noise, not actionable.

# Appendix B — Glossary

| Term | Definition |
| --- | --- |
| AFZ | Automatische Fahrgastzählung — automatic passenger counter (VLAN 8) |
| CCU | Communications Control Unit — Nomad Digital onboard router/gateway |
| DOSTO | Doppelstockwagen — Stadler double-deck passenger trainset |
| FIS | Fahrgastinformationssystem — passenger information system |
| OBS | On-Board Server — Stadler operational platform |
| PWLAN | Passenger Wireless LAN |
| RDC | Remote Diagnostic Computer |
| RSTP | Rapid Spanning Tree Protocol — IEEE 802.1w loop-prevention |
| SFP | Small Form-factor Pluggable — transceiver module on inter-coach trunks |
| ZFR | Zugführerraum — lead driver cab; B1/B3 are the redundant ZFR pair |

# Appendix C — Reference Documents

- ÖBB IPv4 schema: ND-DEL-OBB-035-IPA-138_NV_6Teiler.pdf

- VDS Rail Consist Switch User Manual, version 2.0.4

- Nomad Digital DOSTO L2 Health Check Playbook (project CLAUDE.md)

# Appendix D — Reviewer Notes

Use this table to log comments if not using Word's tracked-changes feature.

| # | Date | Reviewer | Note / Action Required |
| --- | --- | --- | --- |
| 1 |  |  |  |
| 2 |  |  |  |
| 3 |  |  |  |
| 4 |  |  |  |
| 5 |  |  |  |