Onboard Network

L2 Health Check Report

Trainset 4736-109  ·  6-car  ·  DOSTO NEU (Nahverkehr)

Prepared for: Stadler Rail

Date: 2026-05-04  ·  Author: Abbas Rizvi, Nomad Digital

✓ L2 FABRIC HEALTHY — 1 OPEN ITEM FOR STADLER All inter-coach trunks, RSTP, Stadler-facing trunks and firewall path are healthy. One Wi-Fi AP port is down at B3 — requires physical inspection.

# 1.  Document Control

## 1.1  Revision History

| Version | Date | Author | Description |
| --- | --- | --- | --- |
| 1.0 | 2026-05-04 | Abbas Rizvi, Nomad Digital | Initial issue — L2 health check results for Stadler review |

## 1.2  Document Information

| Field | Value |
| --- | --- |
| Document title | 4736-109 Onboard Network L2 Health Check Report |
| Customer | Stadler Rail |
| Trainset | 4736-109 |
| Fzg. ID | 137 |
| Consist type | DOSTO NEU Nahverkehr 6-car |
| CCU hostname | box1-t28 |
| CCU IP | 10.179.28.1 |
| Check date | 2026-05-04 |
| Author | Abbas Rizvi, Nomad Digital |
| Status | Draft for review |

# 2.  Executive Summary

On 2026-05-04, Nomad Digital performed a Layer-2 network health check on trainset 4736-109 (Fzg. ID 137, 6-car DOSTO NEU Nahverkehr). The check covered switch discovery, RSTP topology, per-port error counters, Stadler-facing trunks, ZFR ports, and CCU-to-Stadler-firewall reachability.

The onboard L2 fabric is healthy. All 18 VDS Consist Switches are reachable. All inter-coach trunks are UP at 10 Gbps full-duplex with zero error counters. RSTP has converged on a single root. All Stadler-facing trunks are UP and clean. The CCU-to-Stadler-firewall path is confirmed reachable.

One item requires Stadler's attention: the Wi-Fi Access Point trunk on switch B3 (e0-4) is admin-enabled but has no link. The PoE subsystem confirms the port is powered but drawing 0 W, and the port has never seen any traffic. The AP at car-B3 position appears to not be physically connected to the switch.

## Headline Results

| Check | Result | Detail |
| --- | --- | --- |
| Switches reachable | [object Object] | 18 / 18 VDS switches, 22 Westermo radios |
| Inter-coach trunks (e0-0/e0-1) |  |  |
| Speed | [object Object] | All 32 active trunk ports at 10 Gbps full-duplex |
| Error counters | [object Object] | RX errors / CRC / carrier-false = 0 on all 32 ports |
| RSTP | [object Object] | Single root a0:59:3a:d0:65:20 (D1) agreed by all 18 switches |
| A3 e1-4 — Stadler firewall trunk | [object Object] | UP 1G full-duplex, 0 errors |
| D1/D3 e0-2 — OBS trunks | [object Object] | UP 10G full-duplex, 0 errors |
| D1/D3 e0-3 — RDC trunks | [object Object] | UP 10G full-duplex, 0 errors |
| B1/B3 e1-11 — ZFR ports | [object Object] | UP 1G full-duplex, 0 errors. B3 RX=0 (standby — normal) |
| CCU ↔ Stadler firewall | [object Object] | TCP port 80 + 22 OPEN on 172.19.196.129, ARP REACHABLE, vlan7 0 errors |
| Inter-coach utilisation | [object Object] | Peak ~49 Mbps = 0.49% of 10G capacity |
| B3 e0-4 — Wi-Fi AP trunk | [object Object] | Admin-enabled, link DOWN. PoE on, 0 W drawn. AP not connected. |

# 3.  Scope and Methodology

The check was performed remotely via SSH to the Nomad CCU (box1-t28). All switch commands were issued from the CCU using the standard VDS Consist Switch CLI over SSH with legacy key-exchange algorithms. No configuration changes were made to any device.

## Phases Executed

| Phase | Description |
| --- | --- |
| 1 — Discovery | fping sweep of vlan100 (10.179.28.128/25). ARP table inspected for VDS switches (OUI a0:59:3a) and Westermo radios (OUI 00:14:5a). |
| 2 — Switch fingerprinting | show interface trunks and show vlans run on all switches to identify A3 (firewall), D1/D3 (OBS+RDC), B1/B3 (ZFR) by config fingerprint. |
| 3 — Interface summary | show interface summary run on all 18 switches. Link states and speeds recorded. |
| 4 — RSTP check | show spanning-tree run on all 18 switches. Root bridge MAC and agreement verified. |
| 5 — Error counter scan | show interface <port> details run on all e0-0 / e0-1 inter-coach trunk ports across all 18 switches. |
| 6 — Critical trunk detail | show interface <port> details run on A3 e1-4, D1/D3 e0-2, D1/D3 e0-3, B1/B3 e1-11, B3 e0-4. |
| 7 — Throughput sample | Two byte-counter snapshots 138 s apart on all inter-coach trunk ports. Per-port Mbps derived. |
| 8 — End-to-end FW probe | ARP, vlan7 counters, and TCP probes (port 80 + 22) to Stadler firewall peer on 172.19.196.129. |
| 9 — AP investigation | show interface e0-4 details and show poe interface e0-4 on B3. show log inspected for link history. |

# 4.  Detailed Findings

## 4.1  Switch Inventory

All 18 VDS Consist Switches and 22 Westermo radios were discovered on vlan100 (10.179.28.128/25). Switch count is consistent with a 6-car DOSTO NEU.

| IP Address | Role (fingerprint) | e0-0 | e0-1 | Reachable |
| --- | --- | --- | --- | --- |
| 10.179.28.178 | B1 | 10G UP | 10G UP | [object Object] |
| 10.179.28.179 | — | 10G UP | 10G UP | [object Object] |
| 10.179.28.180 | — | 10G UP | 10G UP | [object Object] |
| 10.179.28.181 | — | 10G UP | 10G UP | [object Object] |
| 10.179.28.182 | — | 10G UP | 10G UP | [object Object] |
| 10.179.28.183 | A3 | 10G UP | 10G UP | [object Object] |
| 10.179.28.185 | — | 10G UP | 10G UP | [object Object] |
| 10.179.28.186 | — | 10G UP | 10G UP | [object Object] |
| 10.179.28.187 | — | 10G UP | 10G UP | [object Object] |
| 10.179.28.188 | — | 10G UP | 10G UP | [object Object] |
| 10.179.28.189 | B3 | 10G UP | 10G UP | [object Object] |
| 10.179.28.190 | — | 10G UP | 10G UP | [object Object] |
| 10.179.28.191 | — | 10G UP | 10G UP | [object Object] |
| 10.179.28.192 | D1 (root) | 10G UP | 10G UP | [object Object] |
| 10.179.28.193 | — | 10G UP | 10G UP | [object Object] |
| 10.179.28.194 | — | 10G UP | 10G UP | [object Object] |
| 10.179.28.195 | D3 | 10G UP | 10G UP | [object Object] |
| 10.179.28.196 | — | 10G UP | 10G UP | [object Object] |

## 4.2  RSTP

RSTP is running on all 18 switches. A single bridge root was elected and agreed upon by all switches. No topology change notifications or root flapping were detected.

| Parameter | Value |
| --- | --- |
| Root bridge MAC | a0:59:3a:d0:65:20 |
| Root bridge IP | 10.179.28.192 (D1) |
| Root bridge priority | 0 (explicitly lowest) |
| Agreement across fleet | 18 / 18 switches |
| Verdict | PASS — single stable root |

## 4.3  Inter-Coach Trunk Error Counters

show interface <port> details was run on e0-0 and e0-1 across all 18 switches (36 port checks total). All ports showed zero on every error counter. The table below shows a representative sample of the active inter-coach trunks.

| Switch | Port | Speed | RX Errors | CRC | Carrier-false | Result |
| --- | --- | --- | --- | --- | --- | --- |
| 10.179.28.179 (—) | e0-0 | 10G | 0 | 0 | 0 | [object Object] |
| 10.179.28.179 (—) | e0-1 | 10G | 0 | 0 | 0 | [object Object] |
| 10.179.28.183 (A3) | e0-0 | 10G | 0 | 0 | 0 | [object Object] |
| 10.179.28.183 (A3) | e0-1 | 10G | 0 | 0 | 0 | [object Object] |
| 10.179.28.192 (D1) | e0-0 | 10G | 0 | 0 | 0 | [object Object] |
| 10.179.28.192 (D1) | e0-1 | 10G | 0 | 0 | 0 | [object Object] |
| 10.179.28.195 (D3) | e0-0 | 10G | 0 | 0 | 0 | [object Object] |
| 10.179.28.195 (D3) | e0-1 | 10G | 0 | 0 | 0 | [object Object] |
| All other 14 switches | e0-0 / e0-1 | 10G | 0 | 0 | 0 | [object Object] |

## 4.4  Stadler-Facing Trunks and ZFR

| Port | Role | Switch IP | Speed | RX Err | CRC | Carrier-false | Notes | Result |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| e1-4 | A3 Firewall trunk | 10.179.28.183 | 1G | 0 | 0 | 0 | TX >> RX: normal (routed traffic) | [object Object] |
| e0-2 | D1 OBS trunk | 10.179.28.192 | 10G | 0 | 0 | 0 |  | [object Object] |
| e0-3 | D1 RDC trunk | 10.179.28.192 | 10G | 0 | 0 | 0 | Low RX bytes — RDC idle/off | [object Object] |
| e0-2 | D3 OBS trunk | 10.179.28.195 | 10G | 0 | 0 | 0 |  | [object Object] |
| e0-3 | D3 RDC trunk | 10.179.28.195 | 10G | 0 | 0 | 0 | Low RX bytes — RDC idle/off | [object Object] |
| e1-11 | B1 ZFR R (active) | 10.179.28.178 | 1G | 0 | 0 | 0 | RX active — B1 is primary ZFR | [object Object] |
| e1-11 | B3 ZFR (standby) | 10.179.28.189 | 1G | 0 | 0 | 0 | RX = 0 — standby, expected | [object Object] |

## 4.5  Throughput — Inter-Coach Trunks

Two byte-counter snapshots were taken 138 seconds apart on all active inter-coach trunk ports. The train was idle (no passengers). Peak utilisation across all 32 active trunk ports was approximately 49 Mbps, which is 0.49% of the 10 Gbps link capacity. No saturation risk.

| Metric | Value |
| --- | --- |
| Sample interval | 138 seconds |
| Active trunk ports sampled | 32 (16 switches × 2 ports; B1/B3 end-of-consist not included) |
| Peak rate (single port) | ~49 Mbps (10.179.28.193 e0-1) |
| Average active trunk rate | ~25 Mbps |
| Peak utilisation | 0.49% of 10 Gbps |
| Baseline vs. Fzg. 146 | Consistent — Fzg. 146 idle baseline was ~140 Mbps per active trunk |
| Verdict | PASS — well within capacity |

## 4.6  CCU ↔ Stadler Firewall Reachability

The Stadler firewall peer on this trainset is 172.19.196.129 (not .1 as in the standard template). This is consistent with the per-train IP allocation for Fzg. 109.

| Probe | Result | Detail |
| --- | --- | --- |
| ARP on vlan7 | [object Object] | 172.19.196.129 — REACHABLE, MAC 00:90:e8:c5:3d:ba (Westermo) |
| vlan7 RX errors | [object Object] | 0 |
| vlan7 TX errors | [object Object] | 0 |
| TCP port 80 | [object Object] | OPEN |
| TCP port 22 | [object Object] | OPEN |
| ICMP to 172.19.196.1 | [object Object] | No route — .1 is not the deployed peer IP on this train. Not a fault. |

# 5.  Open Item for Stadler

The following item was identified during the health check and requires a physical inspection by Stadler. It does not affect the L2 fabric health — all switching, routing and connectivity between onboard systems is functional.

| Field | Detail |
| --- | --- |
| Priority | Medium — no service impact, but Wi-Fi coverage in car B is absent until resolved |
| Switch | B3 — 10.179.28.189 |
| Port | e0-4 (Wi-Fi Access Point trunk, VLAN 100/10/20/30/31/131/150) |
| Observed state | Admin-enabled, line protocol DOWN. PoE mode = on, PoE status = off, power drawn = 0.0 W. |
| Traffic history | RX bytes = 0, TX bytes = 0. The port has never seen any traffic. No link-up events in the switch log. |
| PoE budget | 22 W total drawn out of 202 W available — PoE budget is not a constraint. |
| Assessment | The Wi-Fi AP at car-B3 position is either not physically installed, or the patch cable between the AP and switch B3 port e0-4 has not been connected. The switch is ready to power and serve the AP — the missing element is the physical connection. |
| Required action | Please confirm whether the AP at car-B3 is physically installed. If installed, check that the patch cable is run from the AP to switch B3 port e0-4. If the AP is not yet installed, please advise the expected installation date. |
| How to verify fix | After patching: SSH to 10.179.28.189, run 'show poe interface e0-4'. Status should change to 'on' with a class 4 power draw (~15–18 W). Run 'show interface e0-4' — line protocol should be 'up' at 1000 Mbps. |

Reference — B1 e0-4 (same port, working AP): Switch B1 (10.179.28.178) shows e0-4 UP at 1G — confirming the port config and PoE provisioning are correct on this switch type. The B3 port config is identical; the only difference is the missing physical cable/device.

# 6.  Summary

| Check | Verdict | Notes |
| --- | --- | --- |
| Switch discovery (18/18) | [object Object] | All VDS switches reachable |
| Inter-coach trunk speeds | [object Object] | All 32 active trunks at 10G full-duplex |
| Inter-coach trunk error counters | [object Object] | Zero errors across all 36 port checks |
| RSTP convergence | [object Object] | Single stable root (D1, priority 0) |
| A3 Firewall trunk | [object Object] | 1G UP, 0 errors |
| D1/D3 OBS trunks | [object Object] | 10G UP, 0 errors |
| D1/D3 RDC trunks | [object Object] | 10G UP, 0 errors (RDC idle — expected) |
| B1/B3 ZFR ports | [object Object] | 1G UP, 0 errors; B3 standby (RX=0) — expected |
| Inter-coach utilisation | [object Object] | Peak 0.49% of 10G — no capacity concern |
| CCU ↔ Stadler firewall | [object Object] | TCP 80+22 OPEN, ARP REACHABLE, vlan7 clean |
| B3 e0-4 — Wi-Fi AP | [object Object] | Port admin-enabled, no link, PoE 0W. AP not connected — see Section 5 |

Overall verdict: HEALTHY — the onboard L2 fabric is operating correctly. The single open item (B3 AP not connected) does not affect L2 switching, Stadler-side device connectivity, or passenger information systems.

# 7.  Reviewer Notes

For reviewers who prefer structured written feedback. Add rows as needed. Return to the author before sign-off.

| # | Reviewer | Section | Comment | Resolution |
| --- | --- | --- | --- | --- |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| 3 |  |  |  |  |

End of document  ·  4736-109 L2 Health Check Report  ·  2026-05-04  ·  Abbas Rizvi, Nomad Digital