Network Health Check & Port-Fault Report

DOSTO NEU 6-car trainset – L2 consist fabric and Stadler interconnect

| Vehicle (Fzg. ID) | 146 |
| --- | --- |
| Train number | 4736-118 |
| Monitoring host | 50_6021 (project 50) |
| CCU | box1-t21 (10.179.21.1) |
| Consist | 6-car / 18 VDS Rail Consist Switches (nv6, config v8-146) |
| Assessment date | 19 June 2026 |
| Prepared by | Nomad Digital – Onboard Network Engineering |

# 1.  Executive summary

Nomad Digital assessed the Layer-2 network on vehicle Fzg. 146 (train 4736-118) following three port-down alarms raised in the monitoring system. The assessment covered the consist switch fabric, the three alarming ports, and the Stadler firewall interconnect on VLAN 7.

Overall the L2 fabric and the Stadler interconnect are healthy. The vlan7 addressing is correct, the path to the Stadler firewall is up, and the firewall is confirmed commissioned. The three alarming ports are all administratively enabled and correctly configured on the switch side, with zero error counters — i.e. the consist switches are not at fault. The fault, where present, lies at the far-end device or its cabling.

Of the three ports, one is a confirmed device-down fault requiring action; the other two are members of redundant device pairs whose partner is operational, and we ask Stadler/ÖBB to confirm the intended hot-standby behaviour before classifying them.

## Findings at a glance

| Switch / Port | Connected device | Observation | Classification |
| --- | --- | --- | --- |
| A1 e1-6 | AFZ A2 (VLAN 8) | Admin-up, never linked | FAULT – action needed |
| A3 e1-9 | Sprechstelle A2R (VLAN 9) | Admin-up, never linked; primary (A2) UP | QUERY – redundant pair |
| B1 e1-11 | ZFR primary (VLAN 2) | Admin-up, never linked; standby (B3) UP | QUERY – redundant pair |

# 2.  Scope and method

The following checks were performed against the live train via the onboard Nomad CCU (box1-t21):

- Monitoring review – active alarms for the trainset were read from the network monitoring system (Zabbix), and per-port operational/administrative status was compared against two healthy reference trains of the same build (hosts 50_6023 / Fzg 138 and 50_6018 / Fzg 143).

- Switch identification – each alarming switch was identified to its schema position (A1 / A3 / B1) by its configuration fingerprint and by its own reported system name (e.g. nv6-A1-v8-146), cross-checked against the IPv4 schema (ND-DEL-OBB-035-IPA-146).

- Per-port inspection – show interface <port> details was captured on each alarming switch for link state, auto-negotiation, traffic counters and error counters; the switch event log and uptime were also reviewed.

- Stadler interconnect – the CCU VLAN 7 address was verified against the addressing scheme, and the Stadler firewall was assessed for path health (ARP), commission state (ICMP policy) and service availability (TCP).

# 3.  L2 consist fabric

All three alarming switches are healthy and were up with 2 days 4 hours uptime at the time of assessment, on the expected firmware/config (v8-146). The inter-coach 10 G trunks (e0-0 / e0-1) and the management/AP trunk (e0-4) were up on all three. No CRC errors, carrier-loss events, or runt/giant/jabber frames were observed on the inspected ports.

# 4.  Port-fault detail

Each alarming port is administratively enabled (the switch is actively offering the link) but shows line-protocol down with zero traffic and zero errors in either direction since the switch last booted. This is the signature of a port with no device connected, or a far-end device that is powered off — not a cabling-quality fault (which would show CRC errors, carrier-false events, or runts) and not a switch fault.

## 4.1  A1 e1-6 – AFZ A2  (FAULT)

| Switch | A1 (nv6-A1-v8-146), 10.179.21.204 |
| --- | --- |
| Port / VLAN | e1-6 / VLAN 8 (apc-net) |
| Schema usage | AFZ A2 (10 W PoE device) |
| State | Admin enabled; line protocol DOWN |
| Counters | RX 0 / TX 0 packets, 0 bytes; 0 CRC, 0 carrier-false, 0 errors |
| Reference trains | Same port (A1 e1-6) is operationally UP on Fzg 138 and Fzg 143 |

Assessment: This is a non-redundant device. It is up and carrying traffic on comparable healthy trains, but is absent here. We classify A1 e1-6 / AFZ A2 as a genuine device-down fault. Recommended action: physical check of the AFZ A2 unit and its cabling on car A (power, connector, device health).

## 4.2  A3 e1-9 – Sprechstelle A2R  (QUERY)

| Switch | A3 (nv6-A3-v8-146), 10.179.21.201 |
| --- | --- |
| Port / VLAN | e1-9 / VLAN 9 (call-point-net) |
| Schema usage | Sprechstelle A2R – marked “Redundanz A2” (redundant intercom) |
| State | Admin enabled; line protocol DOWN; 0/0 counters |
| Partner device | Sprechstelle A2 primary (A2 e1-8) is operationally UP |

Assessment: This is the redundant half of an intercom pair; its primary (A2) is up and the intercom function is therefore being served. By the documented Nomad convention a standby member may legitimately be silent. However, on both reference trains this redundant port is operationally UP, so the down state here is a deviation. Query to Stadler/ÖBB: should both members of the Sprechstelle redundancy be link-up (hot standby), or is a silent standby expected?

## 4.3  B1 e1-11 – ZFR primary  (QUERY)

| Switch | B1 (nv6-B1-v8-146), 10.179.21.184 |
| --- | --- |
| Port / VLAN | e1-11 / VLAN 2 (zrp-train-repair) |
| Schema usage | ZFR primary (B3 e1-11 carries the ZFR standby) |
| State | Admin enabled; line protocol DOWN; 0/0 counters |
| Partner device | ZFR standby (B3 e1-11) is operationally UP |

Assessment: The ZFR primary is down while its standby (B3) is up; the ZFR pair shares one address and operates one-active-at-a-time, so the function is being served by the standby. As with the Sprechstelle, the reference trains show this port UP. A separate monitoring alarm (“potential split-brain” on the redundancy controller) is consistent with a primary/standby role split rather than an outage. Query to Stadler/ÖBB: confirm whether the ZFR primary should be link-up alongside the standby, and whether the split-brain indication is expected in this state.

# 5.  Stadler firewall interconnect (VLAN 7)

The CCU VLAN 7 address is 172.19.201.2/17, which is correct for Fzg 146 under the DOSTO NEU addressing scheme (the Stadler firewall peer is 172.19.201.1). The interface is up with clean counters (0 errors, 0 drops).

| Test | Result | Interpretation |
| --- | --- | --- |
| Path (ARP) | 172.19.201.1 REACHABLE (MAC 00:90:e8:…, Westermo) | Path to firewall is up |
| Commission (ICMP) | ping = 100% loss (5/5 dropped) | Firewall commissioned – dropping ICMP by policy as designed |
| Services (TCP) | 80 open, 22 open, 443 timed out | Services reachable; see note below |

Verdict: VLAN 7 is correctly addressed, the path to the Stadler firewall is healthy, and the firewall is commissioned.

Note for Stadler: on the firewall transit address, TCP 80 and 22 answer while 443 is filtered. This does not affect the health verdict, but we ask Stadler to confirm which services 172.19.201.1 is intended to expose, so the open 80/22 ports are recorded as intended rather than incidental.

# 6.  Requested actions

## For ÖBB

- Arrange a physical check of the AFZ A2 unit on car A (A1 e1-6) – confirmed device-down (section 4.1).

## For Stadler

- Confirm the intended hot-standby behaviour for the Sprechstelle redundancy (A3 e1-9 / A2R) and the ZFR redundancy (B1 e1-11): should both members be link-up, or is a silent standby expected? (sections 4.2, 4.3)

- Confirm which services the firewall transit address 172.19.201.1 should expose (TCP 80/22 observed open) (section 5).

- Confirm whether the “potential split-brain” indication is expected while the ZFR primary is down and the standby is active (section 4.3).

Evidence (switch CLI captures, monitoring exports, and reference-train comparisons) is available on request.